GOVERNMENT OF TAMILNADU

TAMIL NADU FIRE & RESCUE SERVICES DEPARTMENT

OPEN  TENDER : 2 OF 2020

TOP SHEET

Tenders are invited from the manufacturer or an authorized distributor/dealer of the manufacturer for supply of the following items as per the specification given in the tender document: -

| Sl. no. | Name of the item | Quantity | EMD |
| --- | --- | --- | --- |
| 1 | Supply of Watermist System and mounting the same on Quick Response Vehicle provided by the Department | 5 | 34,420 |
| 2 | Dewatering Pumps (Diesel Driven Pump to Drain Water About 2.5 lakhs Liters per hour with Trolleys) | 6 | 48,000 |
| 3 | Deep SCUBA Diving Suits | 20 | 33,000 |
| 4 | Borewell Equipment | 20 | 40,000 |
| 5. | Torch lights | 300 | 27,000 |

Note: Please note that it is not necessary that the tenderer should quote for all the items. The tenderer may quote their offer for the items for which they are interested and the EMD paid has to be paid accordingly.

Other details:

| 1. | Address for Communication | Office of the Director, Fire & Rescue Serivices, 17, Rukumani Lakshimipathi Salai, Egmore, Chennai 600 008. Tamil Nadu. |
| --- | --- | --- |
| 2. | Supply of Tender Document | For Down Loading : Tender Document shall be down loaded free of cost at the Department’s Website. By Registered Post: Rs.600/- (inclusive of GST) |
| Note: The tender documents can be accessed from http://www.tnfrs.tn.nic.in and the same can be down loaded free of cost from the web site. The vendors who have downloaded the tender documents from the departmental website must intimate the same through the following email address asupt.tnfrs@gov.in so that any changes can be intimated. Otherwise, it will be considered that the vendor is doing it at his/her own risk. |  |  |

| 3. | Earnest Money Deposit – Mode of Payment. | EMD amount as mentioned above may be furnished as Demand Draft in favour of the Director, Fire & Rescue Services Department, Chennai or specified small savings instruments or irrevocable bank guarantee. Note: EMD should be furnished as mentioned against each item. According to the items for which the tenderer wishes to quote, he has to attach aggregate EMD for the same under relevant Serial numbers. |
| --- | --- | --- |
| 4. | Last date and time for submission of the tender | 21.09.2020 at 14.00 hours. |
| 5. | Date, Time and Place of Opening of the tender. | 21.09.2020 at 14.30 hours at the address mentioned above |
| 6. | Specifications | Furnished in the tender document. Questionnaires have been made as a part of the specification. Answers to be furnished by the tenderer about their offers. |
| 7. | Intimation of changes | Addendum or changes/amendments if any proposed to be issued will be put up in the above web-site. Hence, it is the responsibility of the tenderer to go through the website before submission of their tender. |
| 8. | Type of Tender | TWO COVER SYSTEM - Technical Bid with EMD - Price Bid |
| 9. | Name and address of the party to whom the tender documents sold in person or by post. |  |

| Signature of the Tenderer/s with Date and Seal | Administrative Personal Assistant to DFRS & Deputy Director (Admn)(FAC) Tender Inviting Authority (signature of Tender Inviting Authority not necessary if downloaded from the website) |
| --- | --- |

Note: Each page of the tender form should be signed and to be attached with their tender document.

#### IMPORTANT INSTRUCTIONS TO THE VENDORS

- (a). The vendor shall be the manufacturer of the item/items offered or an authorized distributor/dealer of the manufacturer.

(b) In case he / she is an authorized distributor/ dealer, he / she shall produce along with the tender-offer a certificate of specific authorization from the original equipment manufacturer.

2. (a) EMD to be furnished in the form of  Demand Draft or Fixed Deposit or  Bank Guarantee from any Nationalized Bank in the name of “Director,           Fire and Rescue Services, Tamil Nadu”.

(b) Please note that exemption from payment of EMD under SSI and NSIC   certificates is allowed only if products quoted are covered for manufacturing by the tenderer himself as per the schedules approved by the SSI/NSIC in their certificates issued to the tenderer. If exemption from EMD is sought for, proof of exemption certificate from the  payment of EMD and an undertaking in lieu of EMD should be produced along with sealed tenders, subject to verification from the authorities concerned.

- The inspection of the goods proposed to be delivered will be made  by the officers nominated by Director of Fire and Rescue Services. Hence, if the inspection of goods is proposed to be arranged at manufacturer’s site or any other place than this Department Headquarters at Egmore, the charges for deputing the departmental officers should be borne by the tenderer at his cost.  In view of the above, duly taking into account the above aspects, the price bid furnished by the tenderer should contain the break up details of basic price, GST, freight charges for delivery and such other levies.

- The Tamil Nadu Transparency in Tender Rules 2000 is applicable for the Tender.

5. Mode of submission of tender offer:

(a) Sealed tenders containing:

(i) Technical bid in a separate sealed cover which may be named as “Envelope-A” containing technical details, commercial terms along with a list of the enclosures such as  proof in respect of manufacturer or the authorization letter from the manufacturer, latest local tax clearance certificate if any applicable to the country/origin along with EMD;

and

(ii)  Separate price bid as “Envelope-B” in a sealed cover in the format

prescribed in  the tender form

will be received for and on behalf of the   Tamil Nadu Fire and Rescue Services Department so as to reach on or before the due date prescribed.

(c) The tenderer has the option of sending the offer by Registered Post A/D or by speed post or by Courier  or by air mail for overseas delivery  before the date and time fixed.

(d) Delivery of tender in person will not be accepted on any account and the tender submitted after the last date and time will not be accepted under any circumstances.

(e) The tenderers who could not send their tender by post can put the tender in the Tender Box kept for this purpose at the above mentioned address before the date and time fixed. The slit of the tender box shall be 28 cmx 3 cm and therefore, the cover should be brought of suitable size to move inside the slit.

(f) Any relaxation in the mode of submission of tender will be considered only by the DFRS and his discretion will be final.

Method of submitting price bids:

6. While giving the Price bid, the price details like basic rate (i.e. the rate without taxes and duties) along with separate details of GST, Custom duty, freight charges, Octroi, insurance for delivery, such other levies and delivery charges  that may be applicable must be quoted in the format enclosed.  The total of all these must be separately indicated as NET RATE. Rates should be shown distinctly in words as well as figures. In case of a discrepancy between the rate in words and figures, the lower of the two shall be taken.  If any discount offered by the tenderer, it should be furnished in the Price bid format.

7. The rate shall be valid for 180 days from the date of opening of the tenders.

-0-

Check list for the tenderers in furnishing the tender document:

Envelope A: Technical Bid with EMD

This sealed envelope should contain the following:

- Attach EMD  or exemption proof valid as stated in important instructions to tenderers

- Attach authorization letter obtained from the Original Equipment Manufacturer if you are offering the item  as authorized distribuler/dealer of the manufacturer.

- Attach Technical bid of the offers quoted with Catalogues, Xerox copies of Certificates relevant standard mentioned in the specification  and Test Certificates duly  self- attested.

- Attach a Xerox copy of previous supply orders with proof of supply to various organizations during the last 5 years duly self- attested.

- Attach  General Questionnaire form given in the tender form duly filled and signed

- Attach Specific Questionnaire form given in the tender for each one of the items tendered on technical aspects and other criteria

- Attach the “Undertaking for Execution of Contract” in the form prescribed in the tender document.

- Attach  a copy of GST Registration Certificate, PAN Registration and I.T. Returns filed for the last 3 years

- Attach copy of proofs for payment of GST from 1.4.2019 to 31.3.2020

- Attach the tender form duly signed with seal in each page by the authorized person for having noted the contents.

Envelope B: PRICE BID

This sealed envelope should contain only Price bids  in the format prescribed in the Tender.  All columns should be filled and to be signed with seal by the authorized person.

If different models are quoted, Price bid cover should contain the rates for each model separately in a sealed cover.

OUTER COVER

The outer cover should contain the Sealed Envelopes “A “and “B” and the following address must be written on it:

Deputy Director – Administration,

Tender Inviting Authority,

O/o. the Director, Fire and Rescue Services Department,

17, Rukmani Lakshmipathy Road, Egmore, Chennai-600008,

Tamil Nadu.

The outer cover shall also be sealed and superscribed as “Tender offer for supply  of Watermist Systems and mounting the same on Quick Response Vehicles provided by the Department,  supply of Deep SCUBA Diving Suits,Dewatering Pumps, Borewell Equipment and Torch lights”

SPECIFICATIONS OF THE ITEMS TENDERED

Item No.1: SUPPLY OF WATER MIST SYSTEMS AND MOUNTING THE SAME ON QUICK RESPONSE VEHICLES

(1). General description on the requirement:

The water mist fire fighting systems shall have water tank of capacity at least               550 liters and foam tank of 50 liters and should be designed to be mounted on single cabin Force Kargo King Pick up chassis which shall be provided by this Department.                 The specification details of the vehicle are given below. The Department is intended to use the quick response vehicle on the cab chassis duly fitted with watermist system for the purpose of quick response of fire calls and of various emergencies in narrow urban settlements. It should have minimal requirement of space and weight while optimizing on man power requirement and minimizing the water and other collateral damage to be negligible in comparison with conventional water tender. The unit design shall be complete with opening roller shutter and frame on three sides of the vehicle which provide protection for the unit itself and easy transportation.

(1) General Description for water mist fire fighting system.

The water mist fire fighting system shall be a self - driven fire fighting unit operated with inbuilt petrol engine coupled to a pump and attached with a reducer, hose pipes and  mist generating Guns.The water mist system shall be capable of carrying  550 (Five  hundred and fifty) litres of water and 50 litres of foam concentrate with a separate system to pump and mix in the required ratio, which can be discharged in the form of mist from    a Gun.

The Unit design shall incorporate an external source and filling of water from an external source for direct fire fighting or filling of the tank. The pump should be capable of suction of water from tank or open source and to maintain adequate discharge rate and pressure for operation of nozzle at optimum efficiency. All the water sources to the pump shall be provided with adequate filters and Tank Man hole filling should also consist of filter

The system should have 2 No of 60 meters standard hose reel hose to deliver water to the scene of fire which shall be tested at 1.5 times the max working pressure of the system and the system shall have easy access facility to recoil the hose.

The discharge gun shall be of light weight having stainless steel/brass nozzle with stainless steel filter capable of producing water mist. The gun shall have necessary handles to hold   it properly for fire fighting.

The foam required shall be aqueous film forming foam (AFFF).

The unit shall have adequate provisions for safety of the system

(3) Specification Details of the Unit

Weight of empty 600  liters unit (including tanks, pumping unit, piping, hose reel, etc.) 300 kgs.(Max)

Usable water capacity…..…………………..…..…Min. 550Ltrs.

Built in foam concentrate tank…..………..………Min. 50Ltrs.

No: of deliveries.	………….……..Min. 2Nos.

Main Extinguishing Gun(Water)

Weight of extinguisher gun……………………....2 kgs. Max.

Gun dimension……………………..………….…..As compact as possible.

Working pressure on the nozzle…...... ...............100 bars Max. with nominal  reaction.

Gun operation …....……….......…......................On/Off, Jet, fog

Flow rate……………..........……………........…...Not less than 22 Lts/min(+/- 5%)

Throw……………………………………............. Jet-not less than15m(+/-10%)

Mist-not less than 6m (+/-10%)

Secondary Extinguishing Gun(Foam/water)

Weight of extinguisher gun……………….…….2 kgs. Max.

Gun dimension……………………….………….As compact as  possible.

(The gun shall have an attachment for discharging foam with suitable aeration facility)

Working pressure on the nozzle….....................100 bars Max. with nominal  Reaction.

Flow rate…………….………....…......................Not more than 20 lts/min

Throw…………………………………................Jet-not less than15m(+/-10%)

Mist-not less than 6m (+/-10%)

Hose reel.....................................................2 X 60 mtrs (standard) on manual drum

Max. set operating pressure …………………..100 bars

(4) Main Pump of the Unit ( PLUNGER PUMP)

The ultra high pressure pump should be capable of suction of water from tank, maintain adequate discharge rate and pressure for operation of nozzle at optimum efficiency. The pump shall be imported/reputed make and reliable       /CE marking / or any equivalent in consultation with the department and the same shall be finalized after the approval from the Department.

Max. Pressure…………………………..…..…….100bars

Max. flow	…………………......38  to 40Litre/min.

Make:  UDOR / Fogtec /rosenbauer /firebug /or   any equivalent make

in consultation with department

Note: The supplier must submit OEM/Authorized Dealer Authority Letter   confirming the specifications as mentioned above

(5) Foam Mixing System

A suitable means shall be provided for mixing of foam from tank so that proportion for delivery remains constant and foam quality is maintained.

Size of foam tank……………………………..50 Liter (min.)

Material of tank…………………………….… SS-316 (Stainless steel)

Max.mixing.	………………….…..….6% (Adjustable from 1 to6%)

The foam mixing system should prevent the return of foam inside the water tank and water to the foam tank.

(6) Engine of the Unit

  - The prime mover of pump shall be four stroke petrol driven engine and shall have compatible electrical and lubrication system. The engine should preferably be on 12v Battery for which adequate charger shall also be provided. Suitable electronic instruments shall be provided on the body of engine/panel for fuel level, battery and temperature monitoring. The engine shall have electric starter and emergency manual starter.

Power of the engine………..Not less than 13HP

Fuel…………………….........Unleaded petrol

Starter……………..Electric starter with provision of pull cord over-ride

Level indicator………….….. Mechanical

Engine make ………….Briggs & Stratton/ Honda/ or any other equivalent company in consultation with department.

Note:The supplier must submit OEM/Authorized Dealer Authority Letter   confirming to the specifications as mentioned above and to the satisfaction of the TSC.

(7) Main Water Tank:

The water tank shall be made of SS-304 (stainless steel) material giving a corrosion free life and light weight to the WATER MIST SYSTEM, having usable water storage capacity of 550 liters, properly baffled into segments to reduce water surge. The thickness of the water tank should be of min. 3mm. A man hole of 330mm and overflow pipe and an optical level indicator of water should be part of the tank. The tank must be monolithic (one piece design) to eliminate the problem of the limited mechanical resistance to lateral and centrifugal forces of the tank when vehicle driven on uneven ground.

The   details   of material of tank along with specification  shall be indicated in the offer

(8) Hose Reel Hoses

  - The hoses provided in the system shall be high quality and shall be capable of holding,

The temp ranging from –20 deg C to +55 deg C. The material specification, working pressure, test pressure and cracking pressure should be indicated for each type   of hose. The minimum length of each hose should be 60M.

  - The seals and O-rings used in the system should be   of synthetic material like Nitrite rubber of equivalent.

( 9) GUN

The gun shall be provided with a suitable filter. The discharge rate of main gun shall not be less than 20ltrs/ Min. at pressure not exceeding 100 bars. The design of gun shall be such that there is no appreciable re-coil pressure and the throw shall not be less than 15 mts (Jet Mode) and shall not be less than 6 meters (Spray Mode).The size of mist particle shall be in the range of 150 to 300 microns. The gun shall have an attachment for discharging foam with suitable aeration facility. Nozzles should be CE approved for better consistency and mist quality.

(10) Safety Devices

  - The following safety device shall form part of equipment.

(1) The unit shall have proper heat and exhaust outlet for the engine without causing any interference to the operator while operating the unit.

(2) A suitable pressure reducer with a safety valve, capable of reducing to a low pressure not more than 100 bars & jerk free working pressure shall be incorporated.

  - Any other safety device which is felt to be necessary to be a part of the

  - equipment

(11) BODY WORK

10.1.Enclosed accommodation with two doors for driver and two no’s of crew shall be provided in the Original Chassis .

The structure work of rear body shall be made from 25x25x1.6 mm thick MS square tube welded properly. The complete panelling of the vehicle shall be made from 16 SWG aluminium sheet properly bolted and riveted to super structural members.

10.2 A crew bench seat with good quality cushion shall be fabricated in the rear side of the load body. It shall be placed in front the pump& tank unit. Two number of doors with proper ventilation & weather shield sliding glass shall be constructed in the both side of the load body for easy entry to the crew members.

10.3 The structure work of the rear body shall be provided with rolling shutters on three sides for easy access to the pump. A separate compartment arrangement shall be provided at the underneath of the crew bench seat  for stowage of tools and equipment. The lockers shall have easy and non-struck locking system. All lockers shall be fitted with internal lighting. A master Switch for isolating the locker lighting circuit shall also be fitted in the driving compartment. A Provision shall be made on the top of the vehicle to hold the inflatable boat.  Aerodynamic design shall be made in rear superstructure to avoid the wind resistance. The supplier should submit the design and drawings of the body and stowage before work execution. The design and drawing for rear body and stowage shall be finalized after getting design approval from department officers.

10.3 1 No of grand/Lumax or any equivalent make Flickering LED Beacon bar light head of Red, Blue and White colour with Hooter & P. A. System shall be mounted on top of the driver cabin. 2nos Dual Red-White and 2nos Dual Blue-White Scene-Lighting LED blinkers with inbuilt flasher shall be installed on the upper periphery of the superstructure. 2nos of blue-red combination flickering lights shall be fitted in the radiator grill.

The blinkers shall have an option of minimum 12 watt spot lighting along with blinking for low light visibility. Blinkers and Beacons shall have minimum 6 nos of flash patterns. Blinkers and Beacon shall have Aluminium Base with Polycarbonate Cover. 2 no’s of yellow fog lamps shall be fitted in front bumper of the vehicle.

(12) PAINTING

11.1	The complete super structural members shall be painted with two     coats of Red Oxide primer and two coats of chassis Grey paint.

11.2 The complete external and internal aluminium panelling of lockers etc. shall be painted with two coats of aluminium primer.

11.3 The complete exterior of the vehicle shall be painted with two finish coats of    RAL-3000 colour shade (flame red) reputed brand paint for long lasting glossiness.

11.4 White stripes over red colour body paint shall be painted in both side of the body (model photograph will be issued by the department)

11.5 The Names and logos of the department shall be painted on both sides of vehicle at suitable place in consultation with the Fire Officer.

(1) Accessories 12.1. Electric starter (Enabling the unit to be Electrically started) 1 12.2. Additional hose length of 30 mtrs. with Drum 1 Per set 12.3. Aqueous film forming foam (AFFF) 50 Ltrs. filled in the foam tank and 50 ltrs. spare) 12.4. Tool Kit comprising of 1 no. adjustable spanner 12”, 1 no. cutting plier, 1 set of flat spanners, 1 set of adjustable screw driver, 1 set of suction key and other special tools as per the requirement of the unit.

( 13) Marking & Test

Marking shall give details of capacity, manufacturer’s name, month/ year of manufacturing of the system etc. Test of acceptance of fire fighting system Physical dimensions, performance & weight check of equipment as given under different items above shall form part of test of acceptance.

(14) Operating Conditions.

Water Mist Fire Fighting system shall operate satisfactorily under the following climatic conditions.

- Variation of ambient temperature from–20 deg Ct +55 deg C with 98% relative humidity.

- Dusty and corrosive atmosphere.

- Altitude—upto1200maboveMeanSeaLevel.

(15) Workmanship and Finish

All parts of the appliance shall be of good workmanship and shall have streamlined finish. There shall be no sharp corners around the built area. Necessary beadings and weather strips shall be provided wherever it needs to avoid rain water leakage .Fire service insignia shall be painted on both sides of the appliance in yellow and black lettering.

(16) Test of Acceptance of Fire Fighting System

Physical dimensions, performance and weight check of equipment as given under different items above shall form part of test of acceptance. The appliance shall be inspected in different stages before mounting of the complete unit to the chassis and after mounting of the complete unit on to the chassis. Stage inspections will be carried out from the department during each stage of fabrication. All necessary arrangements shall be arranged by the supplier at the fabrication yard for the proper conduct of such inspections as and when required by the department.

Inspection will be as under:-

a) Finishing inspection.

b) Compliance of specification

c) Water Mist test as per standards

d) Pump Testing & Hose Reel Gun performance

e) Complete functions-operations of all systems

(17) RTO REGISTRATION

RTO registration of the vehicle has already been done by the department. Copy of the Registration documents shall be given by department officers.

(18) WARRANTY

The complete vehicle shall be under warranty for a period of minimum five years from the date of supply of the vehicle against any manufacturing defects. The supplier shall give the guarantee for the supply of spare parts for the super structure, fire pump, engine etc. for the period of 10 years from the date of supply of vehicle.

(19) PICK-UP &DELIVERY

Mounting of Watermist may be carried out by deputing their technicians to our premises where the Quick Response Vehicles which are now kept at the premises of State Training Centre of this Department at Tambaram. If the vehicles have to be taken up to some other workshop for  fitting purpose,  they can take the vehicles at their cost with the consent of our officers.  After completion of the work, it should be stationed at the premises of the State Training Centre or at TFRS head quarters at Egmore, Chennai-8 and the expenditure thereon shall be borne by the tenderer.

(20) COMMISIONING AND TRAINING

The manufacturer's representative shall impart successful commissioning and  the training to the fire brigade personnel regarding operation, handling, use and maintenance of the appliance. Operating manual shall be given along with each vehicle.

Item No: 2:  Specification for Dewatering pump

1.General description on requirement:

Supply of Diesel Driven Pump set comprising of:  ‘MAHINDRA’ or equivalent make model, Water cooled, Three cylinders, Diesel  Engine with  Rated Power of not less than 35  HP (25.7 kW) at 1500 RPM  coupled with ‘KIRLOSKAR’ or equivalent  make model, Self Priming , Non-clogging type pump having 1500 RPM and discharge capacity of  2.5 Lakhs  Ltrs/hour at a total head of 15 mtrs .

2. Capacity of the Unit:

The entire assembly  is mounted on a 2 wheel trolley as per the  specification furnished.

| (1) | Fuel | Diesel driven |
| --- | --- | --- |
| (2) | Horse Power | Rated Power (Gross) 35 HP (25.7 kW) @ 1500 RPM |
| (3) | Discharge capacity | A minimum of 2.5lakhs litres per hour |
| (4) | Suction size | 6” |
| (5) | Delivery size | 6” |
| (6) | Total Head in mtrs | Not less than 15 mtrs |
| (7) | Suction head in mtrs | Not less than 5 mtrs |
| (8) | Weight of the Pump | Not more than1.20 tons |
| (9) | Dimension(L X W X H) | Approx. 2 x 1.5 x 1.5 |
| (10) | Trolley | shall be supplied along with pump as per the specification given below separately |

3. Features:

- Efficiency of the pump offered shall be at par with internationally available pumps

- The Impeller of the Pump shall be Non- clogging to handle suspended solids and is made up of durable material.

- The Pump shall be Portable and easy to transport, when fitted on trolley.

- The Engine shall be suitable to get the maximum efficiency from the impeller.

- The provision of foot valve not necessary, since it is self-priming.  However a strainer with the pump shall be provided.

- Dynamically balanced rotating parts to ensure minimum vibrations.

- Easy maintenance and spares availability: The pump shall be of easy maintenance the spare parts easily available.

- The pump shaft shall be of carbon steel and shaft sleeve is bronze.

- Good quality raw material is used to facilitate longer life with high efficiency.

- The pump is shall be provided with triple delivery outlets with gate valves. It shall  be designed to connect 2.5 inch hoses.

4.Construction of Pump materials:

(1)  Impellor of the pump  shall be made up of durable material.

(2) Delivery casing  shall be made with cast Iron and shaft  is  made up

of carbon steel and shaft sleeve is  bronze.

5. Direction of Rotation:

Direction of Rotation is clockwise viewed from suction side.

6. Each pump set is mounted on a Two wheel  Trolly as per specification below:

(i) Trolley shall constructed with steel channels with strength

(ii) Dimension of the Trolley: Approximate: 8 feet x 4feet

(iii) It shall be mounted on the axis with two tyres size (600 x 16) for easy movement of the trolley

(iv)  It shall be provided with 3 stays made of Iron Rod.

(v)  It shall have the provision of Eye for Toeing with a fire vehicle.

(vi)  It shall have the provision of hose boxes on both sides.

(vii) It shall have the provision of a channel for keeping two length of Suction  hoses each 15feet.

(viii) It shall have the provision for manual pulling bars.

(ix)   Pump and engine shall covered with proper Tin Sheet roof and is detachable

7. ACCESSORIES:

Suction hose ……………………….. 6mtr

Delivery hose ……………………… 15 mtr

Necessary Coupling and Collar shall be provided with the pump set.

8.Certifications:

The seller shall supply the relevant documents to the conformity of the technical specifications  from the OEM of engine and pump manufacturers.

9. Warranty & service

The seller shall give warranty for a period of not less than two years to the entire pump set with all accessories. Also the the supplier shall give assurance to the supply of spare parts for a period of not less than five years.

Item No:3: SPECIFICATION FOR DEEP SCUBA DIVING EQUIPMENT

| S.No | Name of Item | Specification |
| --- | --- | --- |
| 1. | Diving Mask | 1. Tempered single glass,top compartment is equivalent to a dual lens mask |
|  |  | 2. Mineral anti-fogging treated lenses offer a clear field of view. |
|  |  | 3. Chemically resistant silicone skirt |
|  |  | 4. 5 point strap system to enables mask to perfectly fit the face,Stainless steel, quick-adjust, positive-lock buckles for safety |
|  |  | 5. Full face mask should have two second stages attach to it, and communication provision shall be provided. |
|  |  | 6. The mask regulator should be CE marked |
| 2. | Snorkel | 1. Elliptic tube with smooth corrugated flexes connector |
|  |  | 2. Adjustable polycarbonate mouthpiece extensions with purge valve and liquid silicone mouthpiece. |
|  |  | 3. Semi dry deflector to prevents water entering the tube |
|  |  | 4. Easy adjustable snorkel holder for attaching to mask strap |
|  |  |  Should be supplied with one spare of liquid silicone mouth piece with each snorkel |
| 3. | Buoyancy control device. (Size SS-1 S-2, M-3 & L -1 ) | 1. BCD with 1000 Denier outer and 600 denier internal. Hard wearing exterior for abrasion protection and flexible, hard wearing material for inner, for puncture protection and flexibility. |
|  |  | 2. Tank band with durable plastic buckle system for easy to adjust security. |
|  |  | 3. Should have internal octopus holder |
|  |  | 4. 2 side pockets to keep your essentials and extras secure plus restyled graphics and shape |
|  |  | 5. Should provide minimum necessary spares with each BCD |
|  |  | 6. BCD with be CE marked and certificate to EN 1809 standard by EN notified third party. |
| 4. | Regulator and Octopus | First Stageregulator 1 No |
|  |  | 1. First stage reduction valve is to be piston operated type and is set so that the intermediate pressure supplied to the second stage demand valve is maintained at a constant 9-10 bars above ambient sea pressure. |
|  |  | 2. One high and three low-pressure ports with internal inlet filter. |
|  |  | 3. Intermediate Pressure Hose Intermediate pressure hose is to be provided with following length to reach diver's mouth and capable to reduce high pressure to 7-10 bar above ambient pressure. |
|  |  | Second stage regulator, (Diver& Buddy 1+1 No.) |
|  |  | Second stage demand valve to have purge button exhaust tube and finer adjuster. |
|  |  | Mouth piece should be made of silicone or vinyl |
|  |  | Full metal valve housing |
|  |  | Air flow should be at 200 bar: 1400 l/min and Maximum low pressure: 14 bar / 203 PSI |
|  |  | Console |
|  |  | 1. Console with pressure gauge, depth gauge and compass |

| 5. | Hood (S-1, M-4, L-2) | Stretchable 3-6 mm double-lined neoprene with 3 -5 mm smooth face seal and smooth neckband. |  |
| --- | --- | --- | --- |
| 6. | Wet suit (Neoprene) | Men's full suit medium size, raw/cut collar, black/blue colour. |  |
|  | (S- 2, M -3, L- 2) | Made up of 2.5-3 mm neoprene, integrated kneepad, and vertical polymer zipper on back with nylon webbing thumb loop. |  |
| 7. | Hand gloves (S-1, M-4, L-2) | Nylon /neoprene upper atleast 1.5-4 mm, anti slip grip with adjustable wristband and Velcro closure |  |
| 8. | Weight belt | Adjustable nylon belt black/blue colour with plastic buckle for quick release mechanisms |  |
| 9. | Weights | Lead weights for Scuba -should be made up of 100% Lead (H shape)1 Kg each |  |
| 10. | Weight separator | Plastic weight separator suitable for the weight belt |  |
| 11. | Hose/ gauge retainer | Metal quick clip to keeps hoses, gauges, regulators and place the holder anywhere. |  |
| 12. | Boots (Size 7- 2, 8 -2, 9-2, 10 -1 ) | Medium size boots, 3-5 mm neoprene construction with toe& heel cap and polymer zipper. |  |
|  |  | Heavy duty molded sole 3-5 mm sole with anti-slip reinforcement |  |
| 13 | Fin | Open heel type medium size, made up of durable monoprene or plastic, with soft comfort foot pocket. Adjustable buckles and strap made up of high quality and durable rubber or plastic or marine bungee |  |
|  | (S-2, M-3, L-2) |  One spare strap and buckles with each set |  |
| 14 | Diving Knife | 1. Non-corrosive solid 8-9 cm stainless diving knife.A metallic portable sharp knife with a handle, sharp pointed, with straight and serrated cutting edges. |  |
|  |  | 2. Made from 304-grade hard-tempered stainless steel |  |
|  |  | 2.Stainless steel Ozone resistant quick release knife straps |  |
|  |  | 3.Knife should be provided with Quick release sheath. Quick disassembly facility without tools. |  |
| 15 | SCUBA Tank/ Cylinder | 1. ISO certified Nitrox- ready, Aluminum cylinder with standard K Valve. |  |
|  |  | 2. Pressure 3000 psi/200 bar; Capacity 1970- 2190 L, Diameter 175 - 185 mm, Length 160 - 165 mm; Empty wt. 12 – 14 kg, |  |
|  |  |  Should be supplied with 50 spare O-rings |  |
|  |  |  The cylinder drawing will be approved by PESO CCOE Nagpur, cylinder should have PESO, approval as per GAS CYLINDER RULES OF INDIA , Copy of peso approved drawing be submitted with offer. |  |
| 16 | Under Water torch | Rechargeable under water torch |  |
|  |  | Light: LED , Burn time 5-7 hrs in 100 % efficiency, rechargeable battery with charge, Rugged aluminum body, 100 -150 m depth rating. |  |
|  |  | Push-button switch on the end of the light |  |
| 17 | Diving instrument bag | Material- nylon mesh Saltwater-resistant YKK Vision zipper features double-sliders Padded carry handles Volume: 80-90L |  |
| 18 | Scuba Reel | 1. A complete set of reel made up of resistance Aluminium material frame |  |
|  |  | 2. Two positions of friction adjuster for prevent the free rotation. 50-100m polyester line and Stainless Steel Shackle Bolt Snap for Attachment to Gear |  |
| 19 | SCUBA – Diver Tool kit | Portable tool kit complete set for emergency repair of diving gear and equipment |  |
|  |  | Tool in zippered pouch type, with different size and type of Scuba wrenches set, to fit regulator and 1st stages hose attachment, Allen wrench set, Star and flat head screw drivers, needle nose pliers and O-ring tool. |  |

PREQUALIFICATION

1. The bidder will be the authorized distributor of the manufacturer or Dealer of these diving equipment

2. Copy of the letter authorizing them by the OEM / Distributor should be submitted with the offer which should be addressed to the tendering authority for the specific tender.

3. Atleast 100 sets of Diving Gear should be in use in India with various Disaster search & rescue teams / police / fire departments / Navy / Coast Guard supplied in the last 5 years.

4. Supplier should also have after sales service facility and proof of orders executed for service. Copy of these purchase orders should be submitted with the bid.

Item No:4 : Specification for Borewell equipment

- The equipment should rescue the child without any damage to

the life of child.

- The equipment shall access directly on borewell.  The child rescue equipment diameter should be less than 8” inch.

- The equipment should have lifting capacity range of 15-20 kg.

- The equipment should access to a depth of 300ft on the bore well.

- The equipment should consist of a small size, water proof Infrared camera and audio system and portable monitor(operated by battery source).

- The equipment should be convert-able to suit the situation and position of child.

- The equipment shall be operated even by any untrained person.

- The equipment should consist of braking system to lock the child from fall up on further.

- The equipment should be fully controllable in any situation.

Item No.5: Torch light

1. General requirements:-

Type of lamp…………………………………………………LED

Dia of torch light (mm)..………………………….………..101 to 125 mm

Wattage of lamp (Watt)……………………………………35

Type of battery………………………………………………Lithium ion

Type of housing material………………………………….Aluminium

Battery capacity……………………………………………… Not less than 2.6 Ah

Beam angle selection………………………………………. Wide

Front glass quality…………………………………….……..Toughened, scratch resistant

Battery voltage……………………………………………….. 3.6 to 4.2V

Lanyard/ strap………………………………………………… shall be provided

Housing material shall be Aluminium and it shall be rugged, impact and water resistant. Accessories such as Battery charger with cable for external type charging shall be provided along with torch. The Torch light with battery shall pass the drop test from 1 mtr height. The torch light shall be light weight and handy for easy operation.

2. Functional requirements

Light output of torch light………………… Not less than 3200 lumens

Effective distance of torch light………….>/= 400 mtrs

Lamp dimming mode……………………… Step type

continuous light operation time

with maximum lumen output…………… Not less than 5 Hrs

Battery charging time

(with full charging)…….…...................... Not more than 4 Hrs

Operating temperature range…………... -20deg C TO +55 deg C

Operating relative humidity range……… RH 95% at 40 deg C

Lamp life……………………………………….Not less than 50000 Hrs

3.Protection

The torch light shall have Protection against reverse polarity. It shall have protection from battery over discharge and short circuit protection.

IP designation for ingress protection……………………… IP 65

4.Certification details

Test report from Central government/NABL/ILAC Accredited  laboratory to prove conformity to the specification shall be furnished while submitting the offer.

5.Warranty:               Not less than 1 year

Criteria for Evaluation of Tenders

- This is a two-bid tender consisting of :

(i) Technical bid with EMD and

- Price bid

- Technical bid which may be named as “Envelope-A” should contain the technical details of the items offered, commercial terms and conditions along with a list of the enclosures such as proof in respect of manufacturer or the authorisation letter from the manufacturer, latest local tax clearance certificate if any applicable to the country/origin along with EMD.

- Price bid, which should be furnished separately as “Envelop – B” in the format prescribed in the tender form in a sealed cover, and the same will be opened only after evaluation of the technical bid,

- If more than one model is offered, the rate quoted only in respect of the model qualified for selection as per the criteria laid down for evaluation alone will be taken into consideration.  Hence, separate price bid covers may be furnished for each model if more than one model is offered.

- If at any stage, a technical flaw is noticed in following the procedure, the procuring entity will take appropriate decisions and reserve the rights to relax or waive or modify at his discretion any of the condition in the interest of the Department so as to ensure that the overall objective of procurement is not defeated.

- The evaluation of the tender offers will be done in three stages. The rules laid down in Tamil Nadu Transparency of Tenders Act, 1998 and Tamil Nadu Transparency in Tenders Rules, 2000 and other related rules will be scrupulously followed.

Stage I : Rejection of tender offer found technically defective :

- Only the tender document supplied by this Office or the tender documents which were downloaded from the department’s website at free of cost should be used and it should be resubmitted in full shape without detaching any page from it.

- (i) Each page of the tender document, including technical specifications should be duly signed for having noted the contents of the tender document.  Tenders in any forms other than the forms and schedules issued by the department to the tenderer will not considered  and will be rejected as non-responsive.

(ii) Further, the tenderers are requested to put the page number to the tender documents serially including all enclosures while submitting their tender.

- (i) Those who do not submit the EMD, as defined in this tender document, along with the tender (unless the vendor is exempted by a specific rule provision for which documents must be submitted along with the tender to prove that the firm is exempted for this particular item) will not be considered.

(ii) In case of partial amount of EMD, the tender will be rejected straightaway       and the offer will not be considered.  In cases where exemption is sought, a       plea for producing the exemption documents later will not be accepted.

- EMD submitted in a manner other than that specified in the tender document will be considered as not submitted and the tender will be rejected.

- (i) A vendor who does not answer all the questions in questionnaire will not be considered. Only those offers will be considered in which all questions in the questionnaire have been answered.

(ii) Oversight will not be taken as a reason for not answering a question.  It is presumed that a vendor trying for a deal about equipment related with life saving would show adequate seriousness about furnishing details about the product offered by him or not.

- While supplying information in response to the questionnaire answers such as “will be supplied later” or “will be informed later” will not be accepted.  Such answers will lead to the conclusion that the vendor is either not an authorised dealer of the said company or is not serious about the deal.

Stage 2 (Evaluation of Essential Features as per specifications in the tender form):

| 1. | If the tenderer is a manufacturer, attach a proof for the same |
| --- | --- |
| 2. | If the tenderer is not a manufacturer, the offer which is not accompanied with a zeroxed copy of an authorization letter from the manufacturer or some other incontrovertible proof that the vendor is an authorised distributior / dealer of the manufacturer, will be rejected. |
| 3. | An offer, which is not accompanied with a certificate of approval of the standard by the competent authority in respect of the product offered, will be rejected. |
| 4. | The answers given by the tenderer to the questionnaire will be taken into account for verification whether the offer made by the tenderer satisfy the specification and other requirements. |
| 5. | An offer which does not meet out the detailed specifications will be rejected. |
|  | However, the procuring entity will take appropriate decision and reserve the rights to relax or waive or modify at his discretion any of the condition / specification in the interest of the Department, so as to ensure that the overall objective of procurement is not defeated. |

Stage 3 : Selection of the suitable tenders :

The offers that are not rejected during stage 2 above, the technical bids will be further evaluated and finalised based on verification of the sample and the technical details given in the tender document.

Verification /Demonstration of sample

- Sample produced by the tenderers will be verified / tested by the Committee on the date to be specified by the Department in the presence of representative of the concerned tenderer.

In respect of item no:1, the sample of Watermist System offered by the tenderer alone will be verified to ensure its acceptability on performance under technical bid evaluation and before opening of price bid.  After opening of the price bid, the successful tenderer on whom  the purchase order is placed shall carryout mounting their watermist systems on the Quick Response Vehicles provided by this Department at our premises          as given in the specification subject to testing procedures given under clause no.16 of the specification for acceptance towards release for payment.

In respect of item no.2,3,4 and 5  the sample of the offered item should be produced by the tenderer and the same will be verified to ensure its acceptability on performance under technical bid evaluation and before opening of price bid.

The cost for producing the sample for verification should be borne by the tenderer.

- It is the responsibility of the tenderer to keep one sample in readiness at their end and as and when the date and time for verification is fixed and  informed to the tenderer, the same should be produced before  the Committee without fail for verification. Only if any changes in the date and time  fixed for  production of sample, the same will be intimated through proper mode. Further, production of such a sample will, however, be no indication that the items are going to be procured from the vendor.

- In case, any vendor fails to produce the sample on the date fixed without proper reason, it will be presumed that he is unable to produce the sample and his offer will be rejected.  However, the DF&RS or the Committee consider that if the reasons stated by the vendor are reasonable, give more time for production of the sample.

- While evaluation the tender, or after seeing the sample, additional details, if any required may be called for from the tenderers for which a message will be despatched to him through proper mode.  Hence, it is the responsibility of the tenderer to furnish the additional particulars as and when called for.  Furnishing of such additional details will, however, be no indication that it is going to be accepted.  In case, any vendor fails to furnish the details called for within the stipulated time, it will be presumed that he is unable to furnish the details and not interested in his offer and his offer will be rejected.

- While evaluating the technical bids if any one of the tendered equipment does not suit the tender conditions as laid down in technical specifications, the DF&RS reserves the right.:

- To relax or waive or modify at his discretion any of the condition/specification in respect of the equipment in question in the interest of the Department,

(or)

- To reject  all the tenders in respect of the equipment in question without assigning any reasons whatsoever and to suitably modify the numbers to be brought among the selected one of other tendered items to suit our requirement.

Price Bid Opening

- The Price bid should be furnished in the format given in the tender form in a sealed cover as indicated in the tender for each iteml separately.  While giving the Price Bid, the price details like basic rate (i.e. the rate without taxes and duties) along with separate details of GST, Customs Duty, Excise Duty, Freight charges, Octroi, Insurance for delivery and such other levies that may  be applicable must be quoted in the format enclosed.

- The Vendors whose offers have been found to be suitable for the use of this department based on the above evaluation criteria mentioned under stage          3 above  alone will  be called for opening of their price bid at a later date and time fixed by the department which will be intimated to the tender after the technical evaluation of the tender is completed.

- The Price Bid will be evaluated as per the criteria laid down in Tamil Nadu Transparency in Tender Rules 2000 and as amended.

Questionnaire to be filled by the Tenderer :

Note :

- Kindly give to the point and specific answers on a paper or letter head of your Company and put your signatures and seal at the end. In case you use more than one paper or letterhead, please sign and put seal on each page.

- Information given on unsigned papers or letterhead will not be taken into account

- The vendor if he wishes, may, apart from the answers to the questionnaire enclose brochures and leaflets.  However, the information on any pamphlets or brochures submitted by the vendor may be used for corroboration and cross-checking of the information supplied by him or her in answer to the questionnaire but may not be used unless this information is provided in a singed answer-sheet carrying the seal.

- It will be presumed that a vendor who does not answer the questions in this questionnaire is either hiding the information or is not serious about selling his product to the Department.

General Questionnaire

(To be furnished in Letter Head and to be attached with Technical bid )

| Sl.no | Details required |
| --- | --- |
| General Questionnaire |  |
| 1 | The name, address and the e-mail address of the Company. The contact person’s designation, phone number and mobile number |
| 2 | Are you Manufacturer for the offered item or items? If so furnish the details of proof and attach a copy of the same duly self attested |
| 3 | Are you an authorized distributor/dealer of the Manufacturer of the products offered? If so, attach a copy of authorization obtained from the manufacturer |
| 4 | Have you ever been banned by any organizations, if so give furnish the details and attach a copy of the same. |
| 5 | Furnish your GST Registration Number and PAN and attach a copy of the same duly self attested. |

Item No.1: Supply of Water Mist System and Mounting the same on Quick

Response Vehicles

Questionnaire on technical aspects and other criteria

(To be  furnished Letter Head and to be attached with Technical bid)

| Sl.no | Details requested |
| --- | --- |
| 1 | Make and Model of the item offered |
| 2 | Please mention the standard under which the offered item is covered |
| 3 | Whether you have a valid certification obtained from the notified body for meeting the standard given for the offered item and if so attach a copy of the same |
| 4 | Describe the technical aspects for the item offered by you with reference to the detailed specifications given in the tender document under the following heading |
|  | Furnish General description of your offer with reference to para no. 1 of the tender specification. |
| (2) | Describe about the general description of your offered product with reference to para no.2 2 of the tender specification |
| (3) | Describe about the Technical data and features of the Unit, Main Extinguishing Gun (Water), Secondary extinguishing Gun (Foam/Water) with reference to para .no. 3 of the tender specification. |
| (4) | Describe about the Technical data and features of the Main Pump of the Unit (Plunger Pump)with reference to para no. 4 of the tender specification |
| (5) | Describe about the Technical data and features of the Foam Making System with reference to para no. 5 of the tender specification |
| (6) | Describe about the Technical data and features of the Engine Unit with reference to para no.6 of the tender specification |
| (7) | Describe about the Technical data and features of the Main Water Tank proposed to be erected with reference to para no.7 of the tender specification |
| (8) | Describe about the Technical data and features of the Hose Reel System proposed to be provided with reference to para no.8 of the tender specification |
| (9) | Describe about the Technical data and features of the GUN proposed to be provided with reference to para no. 9 of the tender specification |
| (10) | Describe about the Technical features of the Safety Devices proposed to be provided with reference to para no.10 of the tender specification |
| (11) | Describe about the Technical features of the Body Work proposed to be made with reference to para no.11 of the tender specification |
| (12) | Describe about the Painting of the entire unit proposed to be made with reference to ara no.12 of the tender specification |

| (13) | Describe about the Accessories proposed to be provided with reference to para no.13 of the tender specification |
| --- | --- |
| (14) | Describe about your compliance on Operation Conditions as suggested under para no.14 of the tender specification |
| (15) | Describe about your compliance on Workmanship and Finish as suggested under para no. 15 of the tender specification |
| (16) | Describe about your compliance on “Test of Acceptance of Fire Fighting System” as suggested in para 16 of the tender specification |
| (17) | Furnish your compliance of acceptance on “RTO Registration Condition” as suggested in para 17 of the tender specification |
| (18) | Furnish your Warranty clause in compliance to para 18 of the tender specification |
| (19) | Furnish your compliance of acceptance on “Pick up and Delivery” as suggested in para 19 of the tender specification |
| (20) | Furnish your compliance of acceptance on “Commissioning and Training” as suggested in para 20 of the tender specification |
| (21) | Please mention the test report of the competent authority available for the Watermist system proposed to be provided. If so, attach a copy of the same |
| (22) | Whether you have attached a detailed drawing of the body along with other construction features proposed to be made in compliance to the tender specifications attached with the tender. |
| (23) | State clearly if any deviations are required from the specifications given in the tender document for any of the item and if so furnish the justification for the same. What are the alternative suggestions offered by you to meet out the requirement given in the tender specification |
| (24) | Whether any local authorized person or dealer to conduct free check up or to respond in case of any need? Whether the main spare parts will be available in Local/India and how long it will take to supply the spare parts required in case of any need? |
| (25) | Do you undertake to supply the spare parts for the offered item during its life period. If yes submit an undertaking to this effect? |
| (25) | Please attach copies of purchase orders along with invoices for supply of similar items to the Government Department/organizations during the last 5 years. Also indicate the name, address, phone, fax and e-mail address of the organization to whom you have supplied. Please furnish the performance certificate from the user organization if any with your bid |
| (26) | Whether User Manual Instructions booklet will be supplied |
| (27) | Whether you are able to produce the sample of the offered item as and when the date is intimated for verification by the Committee as indicated in the tender document or not? |
| (28) | Please indicate the shortest delivery period required |

Item No.2:  Diesel Driven Pump to Drain Water About 2.5 lakhs Liters

per hour (Dewatering Pumps)  with Trolleys.

Questionnaire on technical aspects and other criteria

(To be furnished in Letter Head and to be attached with Technical bid)

| (1) | Make and model of the offer |
| --- | --- |
| 2 | Please mention the standard under which the offered item is covered |
| 3 | Whether you have a valid certification obtained from the notified body for meeting the standard given for the offered item and if so attach a copy of the same |
| 4 | Describe about the accessories proposed to be supplied with the unit as per the specifications given in the tender document:- |
| (i) | Please mention about General Description of your offered item in compliance to para no.1 of the tender specification |
| (ii) | Please mention about the capacity of your offered item in compliance to sl.no.1 to 10 given under para 2 of the tender specification |
| (iii) | Please mention about the features of your offered item in compliance to para 3 of the tender specification |
| (iv) | Please mention about the construction of pump materials in respect of your offered item in compliance to para 4 of the tender specification |
| (v) | Please mention about the Direction of Rotation capacity in respect of your offered item in compliance to para 5 of the tender specification. |
| (vi) | Please state whether you will provide suitable trolley as described in para 6 of the tender specification |
| (vii) | Please state the accessories to be provided for your offered item in compliance to para 7 of the tendered specification |
| (5) | Please mention the test report of the competent authority available for the offered item. If so, attach a copy of the same |
| (6) | Please mention warranty details, free maintenance and whether training can be offered by you for your product if required |
| (7) | Do you undertake to supply the spare parts for the offered item during its life period. If yes submit an undertaking to this effect? |
| (8) | Whether you have any local authorized person or dealer to conduct free check up or to respond in case of any need? Whether the main spare parts will be available in local/India and how long it will take to supply the spare parts required in case of any need? |
| (9) | Please attach copies of purchase orders along with invoices for supply of similar items to the Government Department/organizations during the last 5 years. Also indicate the name, address, phone, fax and e-mail address of the organization to whom you have supplied. Please furnish the performance certificate from the user organization if any with your bid |
| (10) | Whether you will produce sample for verification by the Committee as and when date is intimated as indicated in the tender document or not. |
| (11) | Indicate the shortest delivery period required |

Item No.3: Deep Diving Suits (SCUBA DIVING EQUIPMENT)

Questionnaire on technical aspects and other criteria

(To be furnished in Letter Head and to be attached with Technical bid)

| 1 | Make and Model of the item offered |
| --- | --- |
| 2 | Please mention the standard under which the offered item is covered |
| 3 | Attach a valid certification obtained from the notified body for meeting the standard given for the offered item. |
| 4 | Describe about the accessories proposed to be supplied with the unit as per the specifications given in the tender document |
|  | Describe about the technical features of Diving Mask of your offered product with reference to para no.1 of the tender specification |
| (ii) | Describe about the technical features of Snorkel of your offered product with reference to para no.2 of the tender specification |
| (iii) | Describe about the technical features of Buoyancy Control Devices of your offered product with reference to para no.3 of the tender specification |
| (iv) | Describe about the technical features of Regulator and Octopus of your offered product with reference to para no.4 of the tender specification |
| (v) | Describe about the technical features of Hood of your offered product with reference to para no.4 of the tender specification |
| (vi) | Describe about the wet suit of your offered product with reference to para no.6 of the tender specification |
| (vii) | Describe about the technical features of Hand Gloves to be provided along with your offered product with reference to para no.7 of the tender specification |
| (viii) | Describe about the technical features of Weight belt of your offered product with reference to para no.8 of the tender specification |
| (ix) | Describe about the technical features of weight aspects of your offered product with reference to para no.9 of the tender specification |
| (x) | Describe about the technical features of Weight Separator of your offered product with reference to para no.10 of the tender specification |
| (xi) | Describe about the technical features of Hose/Gauge Retainer of your offered product with reference to para no.11 of the tender specification |
| (xii) | Describe about the technical features of Boots of your offered product with reference to para no.12 of the tender specification |
| (xiii) | Describe about the technical features of Fin of your offered product with reference to para no.13 of the tender specification |
| (xiv) | Describe about the technical features of Diving Knife of your offered product with reference to para no.14 of the tender specification |

| xv) | Describe about the technical features of SCUA Tank/Cylinder of your offered product with reference to para no.15 of the tender specification |
| --- | --- |
| (xvi) | Describe about the technical features of Under Water Torch of your offered product with reference to para no.16 of the tender specification |
| (xvii) | Whether Diving Instrument bag will be provided along with your offered product with reference to para no.17 of the tender specification |
| (xviii) | Describe about the technical features of SCUBA Reel of your offered product with reference to para no.18 of the tender specification |
| (xix) | Describe about the SCUBA Diver tools kit to be provided with your offered product reference to para no.19 of the tender specification |
| 5 | Please mention the test report of the competent authority available for the offered item. If so, attach a copy of the same |
| 6 | (i) Whether you are Original Equipment Manufacturer of the offered item. If so, whether you have attached proof for the same |
|  | (ii) If you are not OEM, whether you have attached specific authorization obtained from OEM against this tender. |
| 7 | Whether you are fulfilling the criteria of supply of 100 sets of Diving Gear to various Disaster search & rescue teams / Police / Fire departments / Navy / Coast Guard or any other Government Departments in India in the last 5 years. Please furnish copies of purchase orders along with invoices for supply of similar items to the Government Department/organizations during the last 5 years. Also indicate the name, address, phone, fax and e-mail address of the organization to whom you have supplied. Please furnish the performance certificate from the user organization if any with your bid |
| 8 | Please mention warranty details for your product |
| 9 | Please furnish the shortest delivery period required |
| 10 | Whether you have after sales service facility and if so proof of orders executed for service should be submitted with the bid. |
| 11 | Do you undertake to supply the spare parts for the life period of the offered item. If yes submit an undertaking to this effect? |
| 12 | Whether user Manual will be provided |
| 13 | Whether you have any local authorised person or dealer to conduct free check up or to respond in case of any need? Whether the main spare parts will be available in local/India and how long it will take to supply the spare parts required in case of any need? |
| 14 | Whether you will produce sample as and when for production of sample is intimated for verification by the Committee as indicated in the tender document or not. |

Item No.4:  BOREWELL EQUIPMENT

Questionnaire on technical aspects and other criteria

(To be furnished in Letter Head and to be attached with Technical bid)

| 1 | Make and Model offered: |
| --- | --- |
| 2 | Please mention about the standards if any for your offered product |
| 3 | Whether the product offered by you is able to rescue the child without any damage to the life of child. |
| 4 | Whether your offered equipment shall access directly on borewell. Whether the diameter of your offered equipment is less than 8” inch |
| 5 | Whether your offered item has the lifting capacity range of 15-20 kg |
| 6 | Whether your offered equipment shall access to a depth of 300ft on the bore well. |
| 7 | The equipment should consist of a small size, water proof Infrared camera and audio system and portable monitor (operated by battery source) |
| 8 | The equipment should be convert-able to suit the situation and position of child. |
| 9 | The equipment should consist of braking system to lock the child from fall up on further. |
| 10 | The equipment shall be operated even by any untrained person. |
| 11 | The equipment should be fully controllable in any situation. |
| 12 | Please mention warranty details, free maintenance and whether training can be offered by you for your product |
| 13 | (i) Whether you are Original Equipment Manufacturer of the offered item. If so, whether you have attached proof for the same |
|  | (ii) If you are not OEM, whether you have attached authorization obtained from OEM against this tender. |
| 14 | Please furnish the shortest delivery period required |
| 15 | Whether you have after sales service facility and if so proof of orders executed for service should be submitted with the bid. |
| 16 | Do you undertake to supply the spare parts for the offered item during its life period. If yes submit an undertaking to this effect? |
| 17 | Whether user Manual will be provided |
| 18 | Whether you have any local authorised person or dealer to conduct free check up or to respond in case of any need? Whether the main spare parts will be available in local/India and how long it will take to supply the spare parts required in case of any need? |
| 19 | Whether you will produce sample for verification by the Committee as and when the date is intimated as indicated in the tender document or not. |

Item No.5:  TORCH LIGHT

Questionnaire on technical aspects and other criteria

(To be furnished in Letter Head and to be attached with Technical bid)

| 1 | Make and Model of the item offered |
| --- | --- |
| 2 | Please mention the standard under which the offered item is covered |
| 3 | Attach a valid certification obtained from the notified body for meeting the standard given for the offered item. |
| 4 | Describe about the accessories proposed to be supplied with the unit as per the specifications given in the tender document |
| (i) | Whether your offered item is for LED type of lamp |
| (ii) | What is the dia your offered item against the requirement of 101 to 125 mm |
| (iii) | What is the Wattage of lamp as against the requirement of 35 Watt |
| (iv) | Whether your offered is having the Lithium ion type of battery |
| (v) | Whether your offered item is the type of housing material of Aluminium |
| (vi) | What is the capacity of your Battery capacity as against the requirement of not less than 2.6 Ah |
| (vii) | Whether you offered item has wide Beam angle selection |
| (viii) | Whether the Front glass of your offered item has the quality of Toughened and scratch resistant |
| (ix) | What is the capacity your Battery voltage as against the requirement of 3.6 to 4.2V |
| (x) | Whether your offered item is provided with Lanyard/ strap |
| (xi) | Whether your offered item has meeting with the following specification requirements: Housing material shall be Aluminium and it shall be rugged, impact and water resistant. Accessories such as Battery charger with cable for external type charging shall be provided along with torch. The Torch light with battery shall pass the drop test from 1 mtr height. The torch light shall be light weight and handy for easy operation |
| (xii) | What is the capacity of Light output of torch light as against the requirement of not less than 3200 lumens |
| (xiii) | Whether your offered item has the effective distance of torch light………….>/= 400 mtrs |
| (xiv) | Whether your offered item has the Lamp dimming mode of Step type with continuous light operation time with maximum lumen output of not less than 5 Hrs |
| (xv) | Whether your offered item has the Battery charging time (with full charging) of not more than 4 Hrs |
| (xvi) | Whether your offered item has the Operating temperature range of -20deg C TO +55 deg C |
| (xvii) | Whether your offered item has the Operating relative humidity range of RH 95% at 40 deg C |
| (xix) | Whether the Lamp of your offered item has the life not less than 50000 Hrs |
| (xx) | Whether your offered item is meeting the following requirements: The torch light shall have Protection against reverse polarity. It shall have protection from battery over discharge and short circuit protection. IP designation for ingress protection………… IP 65 |
| 5 | Whether you have attached the Test report from Central government/NABL/ILAC Accredited laboratory to prove conformity to the specification along with your offer documents. Please furnish the details. |
| 6 | Please mention warranty details for your offered item |
| 7 | (i) Whether you are Original Equipment Manufacturer of the offered item. If so, whether you have attached proof for the same |
|  | (ii) If you are not OEM, whether you have attached authorization obtained from OEM against this tender. |
| 8 | Please furnish the shortest delivery period required |
| 9 | Whether you have after sales service facility and if so furnish the details |
| 10 | Do you undertake to supply the spare parts during its life period |
| 11 | Whether user Manual will be provided |
| 12 | Whether you have any local authorised person or dealer to conduct free check up or to respond in case of any need? Whether the main spare parts will be available in local/India and how long it will take to supply the spare parts required in case of any need? |
| 13 | Whether you will produce sample for verification by the Committee as indicated in the tender document or not. |

To be attached with Envelope A:

UNDERTAKING FOR EXECUTION OF CONTRACT

Tender for the Supply of Articles

(To be signed by the Tenderer)

To

The Governor of Tamil Nadu,

Represented by

The Director,

O/o the Director of Fire & Rescue Services,

No.17 Rukmani Lakshmipathy Road,

Egmore, Chennai – 600 008.

I, ............................................................  hereby contract and agree on the acceptance of this Tender to supply to the Government of Tamil Nadu (herein after referred to as “the Government”)  in accordance with the conditions of contract stated below, the articles specified in the schedule below of the quality of sort and at the rate of price therein specified and I hereby forward a sum of Rs..................  (Rupees ................................................................ only) as Earnest Money Deposit to be returned to me if this tender is not accepted.

If this tender should be accepted, I hereby undertake to execute within the date specified the department, a contract in accordance with the terms and conditions of contract aforesaid together with such further usual conditions as before the execution of the contract may be agreed upon between the successful Tender and the authority concerned in the Department.

Signature of the Tenderer With Date and Seal

To be attached with Envelope A:

FORM OF  Sales Tax/GST  VERIFICATION TO BE FURNISHED BY THE TENDERER

| Sl.no | Details required | Tenderer’s reply |
| --- | --- | --- |
| 1 | (a) Name and style in which the applicant is registered under GST |  |
|  | (b) GST Registration Number |  |
| 2 | Name and address of all companies’ firms or association or persons in which the applicant is interested in his individual or fiduciary capacity. |  |
| 3 | Total Annual Turn Over in the last three years 2016-17 2017-18 2018-19 |  |
| 5 | Particulars to Sales-Tax/GST paid for the following three years |  |
|  | Year Total T.O. Total Tax Total Tax Balance due Reason assessed assessed paid for balance Rs. Rs. Rs. Rs. 2016-17 2017-18 2018-19 |  |
| 6 | Whether returns were submitted and if so the authority with whom the returns were filed |  |
| 7 | Attach copy of proofs for payment of GST from 1.4.2019 to 31.3.2020. If not attached furnish the reasons for the same. |  |
| 8 | Whether any penal or proceeding for the recovery of sales-tax is pending. |  |

I declare that the above information is correct and complete to the best of my knowledge and belief.

Signature of the  Tenderer with seal

Date:

- CONDITIONS TO BE INCORPORATED IN THE AGREEMENT TOBE MADE WITH THE  SUCCESSFUL TENDERER

1.	This contract is to last for one year certain from its date.  But in the event of any breach of agreement at any time on the part of the contractor, the contract shall be terminated by the Director of Fire and Rescue Services, Chennai on behalf of the Governor of Tamilnadu upon giving fifteen days’ notice to the contractor.

2.	The articles to be supplied under this contract are to be of the quality or sort above mentioned and in every respect equal and answerable to the patterns or submitted during evaluation and such as the Director or any other duly authorised by him shall approve.

3.	The articles are to be delivered free of charge at the contractor’s risk in such quantities or number at such times and in such manner as the Director of Fire and Rescue Services, Chennai or anyone duly authorised by him shall from time to time order.

4.   Rejected articles shall be removed by and at the expense of the contractor within seven days after notice shall have been given him of the rejection.  If not so removed, the Director of Fire & Rescue Services, Chennai may cause the articles to be removed and charge the contractor with all expenses incurred in such removal.

5.	In case of failure by the contractor to deliver articles demanded from him within the period limited for delivery or in case of articles delivered by him not being the stipulated quality, sort, weight or measure or in case of articles being delivered without a correct invoice in triplicate, the Director of Fire & Rescue Services, Chennai or anyone duly authorised by him shall have power to reject any such articles and to purchase others  instead of  any articles so rejected or not delivered unless the contractor shall himself forthwith supply orders that shall be sufficient and satisfactory and any excess cost so incurred  by the Government over the contract price together with all charges and expenses attending the purchase shall be recoverable by the Government from the contactor.

6.	In case of failure to complete the supplies within the date of delivery specified in terms of the contract placed on the contractor the difference between his offer and the next selected offer shall be recoverable by the Government from the contractor.

7.	The contract or any part, share or interest in it is not to be transferred or assigned by the contractor directly or indirectly to any person or persons whomsoever without the written consent of the Director of Fire & Rescue Services, Chennai.

8.	The contractor shall make his own arrangements for obtaining import licences in respect of articles tendered for supply under this contract from the authorities concerned and in no case will his applications in this regard be forwarded.

9.	With every delivery of articles under the contract, the contractor should send invoices or bills in triplicate, which after verification will be passed for payment.

10.	The Government hereby agree that during the term fixed by this contract, and except as herein provided Government shall not purchase from any persons other than the contractor or from any Company or Corporation all or any quantities of the goods or materials agreed to be supplied by the contractor.

11.	Nothing herein contained shall prevent the Director of Fire and Rescue Services, Chennai from at any time requiring the contractor to supply and delivery any larger number of any one or more of the said articles than the number of the same mentioned in the said schedule and in any such case and so often as the same shall happen, the contractor shall and will supply and deliver as aforesaid such additional number of all or any of the said articles subject to the several provisions herein contained as shall be required by him and nothing herein contained shall render it obligatory on the Director of Fire and Rescue  Services, Chennai to take from the contractor the whole or any of the said articles and the contractor shall supply and deliver under this contract only such of the said articles as shall from time to time be required by or on behalf of the Director of Fire & Rescue Services, Chennai.

12.	Any notice to the contractor shall deemed to be sufficiently served if given or left in writing at his usual or last known place of abode or business.

13.	The  contractor shall furnish money security for the due fulfillment of the contract equivalent to       5 percent of the total value of the supplies contracted for.  This security may be in the form of Cash or Demand draft or Bankers’ cheque or specified small savings instruments or where the procuring entity deems fit, irrevocable bank guarantee in a prescribed form.

14.	Upon the complete fulfillment of this contract by the contractor to the satisfaction of the Government or any officer duly authorised by them, the security furnished by the contractor shall be returned to him, less the amount if any due by the contractor to the Government

-0-

OTHER GENERAL TERMS AND CONDITIONS FOR THE TENDER FOR THE SUPPLY OF THE TENDERED ITEMS TO FIRE AND RESCUE SERVICES DEPARTMENT.

Tender Form :

- The Tender schedule is not transferable and only the tenderer to whom it is officially issued by the Fire and Rescue Services Department should use it.

- The Vendors who have downloaded the Tender documents from the Departmental website must intimate the same through the following email address : asupt.tnfrs@gov.in so that any changes can be intimated.  Otherwise, it will be considered that the vendor is doing it at his own risk.

- Only the original tender document supplied by this Office or the tender documents which have downloaded from the department’s website should be used and it should be resubmitted in full shape without detaching any page from it duly signed on each page for having noted the contents of the tender document.  Xerox copies or copies typed on paper will not be accepted and the offer will be rejected.

- Incomplete tender documents will be rejected.

- When the tender is made by an individual, it shall be signed with his name and address and in case of partnership firm it shall be signed by one of the Partners empowered to submit tenders on behalf of the partnership as contemplated.            If it is made by a Company or Corporation it shall be signed by an officer duly authorised for the purpose.  In all cases, the status of the tenderer, whether a Proprietorship, Partnership, Company or Corporation should be furnished in the covering letter with full address of the tenderer’s name, name of the  partners, Directors etc.  The Tenderer should also furnish such particulars / evidence as may be required by the competent authority in this Department before execution of the agreement.

- If any covering letter accompanying the tender document is in the tenderers letter head form, all terms and conditions printed therein should be scored out and duly authenticated.  It should be distinctly understood that such printed terms and conditions in the letter head will not have any binding on this Department and those terms and conditions quoted in the tender document only will be considered.

- No erasers, interpolations, or alterations will be allowed except when initialled by the tenderer.

- The vendor if he wishes, may apart from the answers to the questionnaire enclose brochures and leaflets.

Earnest Money Deposit:

- (a) Earnest money deposit should be paid along with tender by way of Bankers cheque or DD in favour of the Director of Fire & Rescue Services Department, Chennai should be compulsorily appended with the tender document as indicated in the Top sheet of the tender document.  Specified small savings instruments or irrevocable bank guarantee will also be accepted.  Personal / Company cheques will not be accepted.

- Any Tender submitted without EMD will be summarily rejected.

- If exemption from EMD is sought, proof of exemption from payment of EMD from competent authority duly indicating the eligibility for the item mentioned in the tender document along with an undertaking in lieu of EMD should be produced along with sealed tenders.

- In case of unsuccessful tenderers, their Earnest Money Deposit will be returned to them in due course soon after the final decision is taken on the tenders by the department.  The Earnest Money of unsuccessful tenderers outside Tamil Nadu State will, however, be returned by money order or by DD and the money order commission or DD Commission will be deducted from the amount refundable.  The Earnest Money Deposit will not bear any interest.

Prices : Details to be furnished and conditions :

- (a) Rates should be shown distinctly in words as well as figures.

(b) Prices quoted should include the cost of all accessories essential to make the      equipment/article fully functional.

(c) The price bid should be furnished as mentioned in the “Important instructions       to the Vendor” given in the tender document.

(d) If a foreign vendor is willing to participate in the tender directly, he should         furnish his offer as mentioned in the “Important instructions to the Vendor”       given in the tender document.

(e) Further, the tenderer shall agree to extend the validity of the tender without       altering the substance and prices of their bid for further periods, if any,       required by the purchaser.

(f) No charges other than those mentioned in the supply order will be paid under      any circumstances.

Demonstration of equipment and Evaluation :

- (a) The evaluation of tenders will be made as per “Criteria for evaluation of       tenders’ given in the tender document.

(b) The Tenderer should produce the equipment/materials which he has quoted       in the tender document for demonstration before the evaluation committee on       the date fixed for production.  The tender will rejected if the article is not       produced for demonstration and Earnest Money Deposit is liable to be      forfeited in the event of such failure.

(c) Each sample produced for demonstration must have a label attached to it,       showing clearly the name fo the tenderer and the country of manufacture.

# Warranty

12 (a)	The warranty period shall be as may be stipulated for the article concerned from the date of supply.

(b) The warranty shall cover defects in materials or bad workmanship under  normal use and service for the period of warranty notwithstanding the fact that the material were inspected and approved by inspecting officers.

# Acceptance of tender

13 (a) The lowest tender will not necessarily be accepted.

(b) Notification of acceptance of tender by the Department shall be deemed to result in and constitute a valid and considered contract binding on the tenderer.  If the tenderer withdraws  his / her tender after acceptance by the Department, the EMD shall be forfeited.

Release of Purchase Order.

14.  The contractor should examine carefully the purchase order immediately on receipt and bring to the notice of the indenting officer within five days of receipt of purchase order and discrepancy with regard to the quantity ordered, specifications, packing, etc., for prompt rectification.  The letter should be sent by registered post Acknowledgement due.  Meanwhile, supply of the materials, which are cleared in all respects for despatch and acceptance by the consignees shall be arranged by the date mutually, agreed upon and should not be held up for any reason whatsoever.

# Security Deposit

15(a)	Security Deposit equivalent to 5 percent of the total value of the supplies contracted for will have to be furnished by the successful tenderer.   This security may be in the form of Cash or Demand draft or Bankers’ cheque or specified small savings instruments or where the procuring entity deems fit, irrevocable bank guarantee in a prescribed form.    This is liable to be forfeited either in full or in part for any breach of conditions of contract or default on the part of the contractor.

(b)If the successful tenderer fails to  remit the Security Deposit and execute the agreement within the period specified the Earnest Money Deposit will be forfeited  The security deposit will be refunded only upon satisfactory fulfillment of the contract after completion of supply of the entire materials ordered and  the same can be adjusted as against furnishing of performance guarantee after supply.

(c)  Security Deposit paid by the successful tenderer will not bear any interest.

Agreement

16 (a)	The successful tenderer should execute an agreement in non-judicial stamp paper, having a face value of Rs.100/- or any other value intimated in the provisional order within the date specified therein in the Form, which can be seen at the Office of the Director of Fire & Rescue Services, Chennai, for the due fulfilment of the contract.  The Stamp duty on the agreement shall be borne by the successful tenderer.

(b) On their executing the agreement, the Earnest Money will be returned to them.  In case firms outside Tamil Nadu State the EMD will be returned by money order or by DD and the money order commission or DD commission will be deducted from the amount refundable

# Delivery

17 (a)The rates accepted will be for delivery at such destinations as indicated in the tender document or in subsequent purchase order.

(b)The delivery period should be mentioned by the Tenderers in their offers.  However, the maximum time limit for supply of the goods will be fixed by the Director while executing the agreement with the successful tenderer. The supplier shall reckon the delivery period from the date of receipt of the purchase order.  The date of delivery as per postal acknowledgement due in the case of purchase order sent by post or the date of delivery if delivered in person or through special messenger shall be deemed to be the date of receipt of the purchase ordered.

(c )Delivery period is inclusive of pre-delivery inspection.  The date if delivery of materials in good condition to the consignees shall be the date of supply.  The rate of progress indicated in the purchase order should be maintained.  The materials should be delivered to the consignees as per the purchase order placed.

Acceptance of materials by the consignee:-

18. (a) If on examination of the items supplied, any portion of the supplied materials / equipment is found to be damaged or substandard or not fully in accordance with the relevant specifications, the whole supply  shall be rejected irrespective of the fact that this material was cleared under pre-delivery inspection.  The materials / equipment should strictly conform to the specifications given in the tender conditions.  In case of doubt whether the materials conform to the specifications or not, the decision of the consignee shall be final.  The consignee has right to reject the equipment found not conforming to the specifications and they should be replaced within seven days at the cost of the supplier.

(b) The item/items supplied by the successful bidder shall clearly indicate in the form of a permanent label or in some other indelible form such details as batch no. Serial No. Year of manufacture, name of the manufacturer etc to authenticate that the given item has been actually manufactured by the said manufacturer.

# Penalty: -

19 (a) Where their supply is not completed within the delivery schedule, without prejudice to the right of the Department to cancel undelivered portion of the purchase order, extension of time may, if so required by the contractor may  be granted at the discretion of this Department.  Such extension of time will entail recovery of penalty from the contractor at one percent per week, on the value of the undelivered quantities subject to a maximum of five percent of the value of the undelivered quantity.  However, the Director may waive the penalty on valid reasons as per his discretion. .

(b)The officer authorised by this Department will have the right to cancel the purchase order and to recover from the contractor liquidated damages at 2% of the value of the undelivered quantity or the extra financial commitment to this Department on the purchase of the balance quantity, whichever is higher besides forfeiting the security deposit.

Performance guarantee

20. After successful completion of the items, 10 percent of the   payment amount will be retained during the warranty period as performance guarantee.  Alternatively, if the tenderer gives an irrecoverable Bank Guarantee in lieu of the balance 10 percent, the department reserves the right to accept the same. If the warranty period exceeds 2 years, the minimum period for which performance guarantee to be given and the alternative method to ensure the warranty period exceeding 2 years will be decided during the negotiation with the successful tenderer.  On receipt of Performance Guarantee the Security Deposit paid will be refunded/discharged.

Payment:

21.On successful completion of supply to the satisfaction of the Fire & Rescue Services Department and on receipt of the invoice bill in triplicate, 100% payment will be settled immediately.  If the tenderer is from abroad, payment terms for imports shall be based on standard terms of international trade and the payment will be effected through irrecoverable letters of credit drawn on banks as per the rate fixed as per negotiation.

Others:

22.	The tenderer should furnish details of orders executed for Government Departments / Departments / autonomous boards etc., and details of orders on hand or pending execution.  Certificates in proof of these should also be furnished.

23.	The Director of Fire & Rescue Services Department, Chennai, acting on behalf of the Governor of Tamil Nadu or any other authority empowered in this matter shall have the right of rejecting any tender without assigning any reason for doing so.

24.	The proposed quantity to be procured in respect of each item is mentioned in the tender document in relevant page.   However, the Director of Fire & Rescue Services Department, Chennai or any other authority empowered shall have the right or requiring a successful tenderer to supply a greater number or reduce the number than the number or quantity mentioned in the schedule.

25. Tenders will be liable to rejection for the following reasons apart from any other    reasons mentioned anywhere else in this document.

  - If the country of manufacture and of the origin of material used in manufacture of articles is not noted in the tender.

  - If the tender is not in the form supplied by this Office or downloaded from the departmental website.

  - If the offer is not as per the specifications mentioned in this tender document.

  - If samples of / models of articles quoted are not submitted as and when required for technical evaluation test.

  - If the tender is not accompanied by the receipt of Earnest Money.

  - If the tenderer enters any restriction on other conditions in his tender.

  - If the tenderer enters one rate in figures and another in words and declines to abide by the lower of the two.

  - If the tenderer alters the period of supply or expunges any clause in the form of tender, and

  - If the tender is presented without signature.

# Reservation of rights

26.	The authority competent reserves the rights:-

(1) The Tender Inviting Authority may extend the last date and time for receiving tenders after giving adequate notice to all intending tenderers in cases where:-

- the publication of the tender notice has been delayed

- the communication of changes, in the tender documents to the prospective tenderers took time

- any of the tenderers requested clarifications communication of which took time to all the tenderers and

- any other reasonable grounds exist, for such extension which shall be recorded in writing by the Tender Inviting Authority.

(2) If at any stage, a technical flaw is noticed in following the procedures, the procuring entity will take appropriate decisions to ensure that the overall objective of procurement is not defeated.

(3) After negotiation with the tenderer and before passing the order accepting a tender, if the Tender Accepting Authority decides that the price quoted by such tenderer is higher by the percentage as may be prescribed over the schedule of rates or prevailing market rates, the tender shall be rejected.

(4) The Tender Accepting Authority before passing the order accepting a tender, may also reject all the tenders for the reasons such as changes in the scope of procurement, lack of anticipated financial resources, court orders, accidents or calamities and other unforeseen circumstances.

(5) The purchaser reserves the right to:-

  - vary the quantity finally ordered to the extent of 25% indicated in the Tender document.

  - To split the tender quantity and place orders on one or more than one firm to meet the delivery requirements.

  - Recover losses, if any, sustained by this Department, from the supplier who pleads his inability to supply and backs out of his obligation after award of contract.   The EMD and the Security Deposit paid shall be forfeited.

  - To cancel the orders for not keeping up the delivery schedule.

(v)    To award contract to one or more tenderers in respect of each item covered by the tender.

(vi)     To reject any or all the tenders without assigning any reasons therefor.

(vii)    To relax or waive or modify at his discretion any of the condition /

specification in the interest of the Department.

(viii)  The decision of the tender accepting authority  in this regard shall be   final,  conclusive and binding on the supplier.

# Certificates and Details to be furnished

27.	GST Registration and PAN Registration should also be appended with the tender document.

28.  The following certificates may also be furnished:

- Valid NSIC Registration Certificate, if registered under NSIC

ii.         Valid SSI Registration Certificate, if registered under SSI

iii.        Valid ISO 9001 Registration if any.

## CERTIFICATE BY THE TENDERER

I / we have gone through the terms and conditions and will abide by them as laid down above and in other sections of this Tender document.

I / We hereby declare that the particulars furnished by us in this offer are true to the best of my / our knowledge and we understand accept that, if at any stage the information furnished are found to be incorrect or false, we are liable for disqualification from this tender and also liable for any penal action that may arise due to the above.

Signature of the Tenderer

with  date and Seal.

| PRICE BID FORMAT FOR “ Supply of Watermist System and mounting the same on Quick Response Vehicle provided by the Department ” ( Note: To be furnished in “ ENVELOPE –B” ) This format may be Xeroxed for furnishing price bid for different make/ models. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | i) Description of article ii) Make and Model iii)capacity iv) Country or origin of the article or components (Note:Separate price bid should be given for each make/model if more than one make/model of any of the item) | Basic price (In case if the tenderer is from abroad, the prices shall be quoted either in U.S.Dollars, Pounds, Euro or any other freely convertible currency) | GST in % and HSN Code | Other taxes,levies, charges including Customs duty, Excise duty and transportation charges etc. (specify the type of tax/charge and the amount for each such item) | Total including GST and levies and any other charges | Quantity as per tender | Grand Total for entire quantity including GST, levies and any other | Please mention if any discount is offered | Other details (FOR terms, Payment terms, Validity and Delivery Schedule should be furnished in this format) |

Note:  Please answer the questionnaire:

(i) Whether you are quoting concessional GST? if so, mention authentication for the same.

(ii) If the authentication mentioned by you is not acceptable one, what will be the tax component to be taken into account                                        in evaluating the price bids?

Signature of the Tenderer with seal

| PRICE BID FOR SUPPLY OF “Dewatering Pumps (Diesel Driven Pump to Drain Water About 2.5 lakhs Liters per hour with Trolleys) ( Note: To be furnished in Envelope B) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | i) Description of article ii) Make and Model iii)capacity iv) Country or origin of the article or components (Note:Separate price bid should be given for each make/model if more than one make/model of any of the item) | Basic price (In case if the tenderer is from abroad, the prices shall be quoted either in U.S.Dollars, Pounds, Euro or any other freely convertible currency) | GST | Other taxes,levies, charges including Customs duty, Excise duty and transportation charges etc. (specify the type of tax/charge and the amount for each such item) | Total including GST and levies and any other charges | Quantity as per tender | Grand Total for entire quantity including GST, levies and any other | Please mention if any discount is offered | Other details (FOR terms, Payment terms, Validity and Delivery Schedule should be furnished in this format) |

Note:  Please answer the questionnaire:

(i) Whether you are quoting concessional sales tax? if so, mention authentication for the same.

(ii) If the authentication mentioned by you is not acceptable one, what will be the tax component to be taken into account                                        in evaluating the price bids?

Signature of the Tenderer with seal

| PRICE BID FORMAT FOR SUPPLY OF “DEEP SCUBA DIVING EQUIPMENT” ( Note: To be furnished in “ ENVELOPE –B” ) This format may be Xeroxed for furnishing price bid for different make/ models. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | i) Description of article ii) Make and Model iii)capacity iv) Country or origin of the article or components (Note:Separate price bid should be given for each make/model if more than one make/model of any of the item) | Basic price (In case if the tenderer is from abroad, the prices shall be quoted either in U.S.Dollars, Pounds, Euro or any other freely convertible currency) | GST | Other taxes,levies, charges including Customs duty, Excise duty and transportation charges etc. (specify the type of tax/charge and the amount for each such item) | Total including GST and levies and any other charges | Quantity as per tender | Grand Total for entire quantity including GST, levies and any other | Please mention if any discount is offered | Other details (FOR terms, Payment terms, Validity and Delivery Schedule should be furnished in this format) |

Note:  Please answer the questionnaire:

(i) Whether you are quoting concessional sales tax? if so, mention authentication for the same.

(ii) If the authentication mentioned by you is not acceptable one, what will be the tax component to be taken into account                                        in evaluating the price bids?

Signature of the Tenderer with seal

| PRICE BID FORMAT FOR SUPPLY OF “ Borewell Equipment” ( Note: To be furnished in “ ENVELOPE –B” ) This format may be Xeroxed for furnishing price bid for different make/ models. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | i) Description of article ii) Make and Model iii)capacity iv) Country or origin of the article or components (Note:Separate price bid should be given for each make/model if more than one make/model of any of the item) | Basic price (In case if the tenderer is from abroad, the prices shall be quoted either in U.S.Dollars, Pounds, Euro or any other freely convertible currency) | GST | Other taxes,levies, charges including Customs duty, Excise duty and transportation charges etc. (specify the type of tax/charge and the amount for each such item) | Total including GST and levies and any other charges | Quantity as per tender | Grand Total for entire quantity including GST, levies and any other | Please mention if any discount is offered | Other details (FOR terms, Payment terms, Validity and Delivery Schedule should be furnished in this format) |

Note:  Please answer the questionnaire:

(i) Whether you are quoting concessional sales tax? if so, mention authentication for the same.

(ii) If the authentication mentioned by you is not acceptable one, what will be the tax component to be taken into account  in evaluating

the price bids?

Signature of the Tenderer with seal

| PRICE BID FORMAT FOR SUPPLY OF “ TORCH LIGHT” ( Note: To be furnished in “ ENVELOPE –B” ) This format may be Xeroxed for furnishing price bid for different make/ models. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | i) Description of article ii) Make and Model iii)capacity iv) Country or origin of the article or components (Note:Separate price bid should be given for each make/model if more than one make/model of any of the item) | Basic price (In case if the tenderer is from abroad, the prices shall be quoted either in U.S.Dollars, Pounds, Euro or any other freely convertible currency) | GST | Other taxes,levies, charges including Customs duty, Excise duty and transportation charges etc. (specify the type of tax/charge and the amount for each such item) | Total including GST and levies and any other charges | Quantity as per tender | Grand Total for entire quantity including GST, levies and any other | Please mention if any discount is offered | Other details (FOR terms, Payment terms, Validity and Delivery Schedule should be furnished in this format) |

Note:  Please answer the questionnaire:

(i) Whether you are quoting concessional sales tax? if so, mention authentication for the same.

(ii) If the authentication mentioned by you is not acceptable one, what will be the tax component to be taken into account

in evaluating the price bids?

Signature of the Tenderer with seal