GOVERNMENT OF TAMILNADU

TAMIL NADU FIRE & RESCUE SERVICES DEPARTMENT

Tender Notice No :  03 of 2020-21

GLOBAL   TENDER

Tender document for purchase of one Aerial Ladder Platforms of

Not less than 54 m

TOP SHEET

Tenders are invited from the manufacturer or an authorized distributor/dealer of the manufacturer for supply of the following items as per the specification given in the tender document:-

| Sl.no. | Name of the item | Quantity | EMD |
| --- | --- | --- | --- |
| 1. | Aerial Ladder Platforms of Not less than 54 m | 1 | 12,00,000/- (Rupees Twelve lakhs only) |

Other details:

| 1. | Address for Communication | Office of the Director, Fire & Rescue Serivices, 17, Rukumani Lakshimipathi Salai, Egmore, Chennai 600 008. Tamil Nadu. |
| --- | --- | --- |
| 2. | Supply of Tender Document | For Down Loading : Tender Document shall be down loaded free of cost at the Department’s Website. Tender cost to obtain in person: Rs.590/- ( inclusive of GST) By Registered Post: Rs.690/- (inclusive of GST+Postal charges) |
| Note: The tender documents can be accessed from http://www.tnfrs.tn.nic.in and the same can be down loaded free of cost from the web site. The vendors who have downloaded the tender documents from the departmental website must intimate the same through the following email addresses ddhq.tnfrs@gov.in, asupt.tnfrs@gov.in so that any changes can be intimated. Otherwise, it will be considered that the vendor is doing it at his/her own risk. |  |  |

| 3. | Earnest Money Deposit – Mode of Payment. | EMD amount as mentioned above may be furnished as Demand Draft or specified small savings instruments of Fixed Deposit or irrevocable bank guarantee in favour of the Director, Fire & Rescue Services Department, Chennai |
| --- | --- | --- |
| 4. | Last date and time for submission of the tender | 09.02.2021 at 14.00 hours. |
| 5. | Date, Time and Place of Opening of the tender. | 09.02.2021 at 14.30 – 15.00 hours at the address mentioned above |
| 6. | Specifications | Furnished in the tender document. Questionnaires have been made as a part of the specification. Answers to be furnished by the tenderer about their offers. |
| 7. | Intimation of changes | Addendum or changes/amendments if any proposed to be issued will be put up in the web-site from http://www.tnfrs.tn.nic.in. Hence, it is the responsibility of the tenderer to go through the website before submission of their tender. |
| 8. | Type of Tender | TWO COVER SYSTEM - Technical Bid with EMD - Price Bid |
| 9. | Name and address of the party to whom the tender documents sold in person or by post. |  |

| Signature of the Tenderer/s with Date and Seal | Signature of the Tender inviting authority (not necessary if downloaded from the website) |
| --- | --- |

Note: Each page of the tender form should be signed and to be attached with

the tender document.

Check list for the tenderers in furnishing the tender document:

- Sign the tender form in each page and attach the same with your tender document.

(2) Attach EMD as described in the tender form along with technical bid in a separate sealed cover which may be named as “ Envelope-A”

- If you are the manufacturer of the item quoted attach proof for the same.             If you are an  authorized distributor/ dealer, you should produce along with the tender offer a certificate of specific authorization from the original equipment manufacturer in Envelope-A.

- Attach Questionnaire form given in the tender form duly filled along with your tender document in Envelope-A.

(5)  Attach Price bids only in the format prescribed in the tender form in  a separate sealed cover as “Envelope-B”.

- As mentioned in the tender specification, the tenderer should depute a responsible person on the date of opening of the technical bids, who should be able to make a video presentation about their offer before the Tender Evaluation Committee on the same day  after opening of the technical bids.   Further, the tenderer has to produce a C.D. duly authenticated by him before the Committee.

- The following address must be written on the OUTER ENVELOPE:

Deputy Director –Administration (F.A.C)

O/o. the Director, Fire and Rescue Services Department,

17, Rukmani Lakshmipathy Road, Egmore, Chennai-600008,

Tamil Nadu, India.

This cover shall be superscribed Tender offer for supply of “AERIAL LADDER PLATFORMS OF NOT LESS THAN 54 METERS”.

SPECIFICATIONS FOR FABRICATION AND SUPPLY OF AERIAL LADDER PLATFORMS OF NOT LESS THAN 54 METERS WORKING HEIGHT FOR THE USE OF    TAMIL NADU FIRE AND RESCUE SERVICES DEPARTMENT

1. GENERAL REQUIREMENT:

1.1	The Aerial Ladder Platform (ALP) shall be designed specifically for the purpose of Fire Fighting and Rescue. ALP should comprise  a  main  boom  with  Telescopic  sections  and Articulated Boom / s with cage mounted at the end of the boom and the entire unit shall be mounted on a Turn Table on a Heavy Duty Diesel Engine chassis of VOLVO make, for having a minimum of  300 hp with appropriate Wheel Base with  fully  factory  built  cabin  and  suitable  capacity  PTO.  The Vehicle Chassis shall comply with Euro VI emission norms.

1.2 	The operational stability and structural strength based on the criteria laid in EN  1777 with appropriate norms for load bearing steel structures for mobile hydraulic appliances. Other norms and standards  applicable  for  elevated  raised  Platforms  used  for  Fire  Fighting  and Rescue  operations. Certificate  to  that  effect  issued  by  the  competent agency shall be enclosed with the Tender. The Vehicle shall be CE marked.

1.3 	The ALP shall be capable of use at any angle of elevation without any reduction of load capacity of the cage. It should rotate 360 degree at any angle of  elevation  as  well  as  below  ground. However, it should be ensured that the  boom  shall not touch any portion of the vehicle body or any obstructions during the above operations.

1.4 	The appliance shall be compact and fast on the road and easily maneuverable in the crowded streets and around sharp corners.

1.5 The overall dimensions shall not exceed the limits specified in point 3.

1.6 	A  Telescopic  Rescue  Ladder  shall  be  attached  on  the  right  hand  side  of  the booms.  The  ladders  shall  be  provided  with  sufficient  width  and  handrails  for rescue of the people at any height during Rescue operation.

1.7 	The  ALP  shall  be  computer electro  hydraulically  controlled,  permitting  precise  and  easy  operations  under  the  most  difficult  conditions,  with  ample reserve strength and stability.

1.8 	Full safety interlocks shall be incorporated in the design so as to ensure complete safety in operations and long years of reliable and trouble free service. The system should be fail proof.

1.9 Even in extreme operating and climatic conditions it should be fail proof. The safe working loads ratings shall include an allowance for the weight of water system and the reaction from the monitor jet while operation.

1.10 	The vehicle shall have a leveling system to adjust axial and transverse movement to an angle not less than 7 degrees and it shall be in automatic in nature.

1.11	In addition to the above, there  shall  be  a  full  back  up  system  for  all  boom  movements  and  outrigger movement.

1.12. The Control system of the Aerial Ladder Platforms shall be fully tropicalised and be able to operate in the temperature range up to +60 degree centigrade and in a dusty and Humid condition without reducing the maximum operation limits.

2. CHASSIS:

2.1	The Chassis shall be of VOLVO make 6x4; having a minimum of 400 hp with appropriate wheel base as per Central Motor Vehicle Rules 1989 with full factory built cabin and suitable capacity PTO. The vehicle Chassis shall be right hand Drive and shall comply with BS VI (Euro VI) emission norms.

2.2	The Chassis shall be homologated from the appropriate authority in India in case not already an approved model.

2.3	The engine shall be six cylinders, inline, Diesel with direct injection, turbo charged with intercooler.

2.4	The displacement of the engine shall cater to the need of developing adequate horse power.

2.5	The clutch shall be single plate, dry type, and power assisted Automatic type.

2.6	The gearbox shall be nine speed synchronized with crawler gear.

2.7	Rear Axle shall be Tandem Bogie type with Hub reduction and differential lock between the wheels and axles.

2.8	Chassis frame shall be ‘C’ Channel section made of high strength steel with cross members.

2.9	The Steering shall be integral power steering with collapsible steering wheel and column.

2.10	The Front Suspension shall be leaf spring type and the rear suspension shall be reverse s camel type with shock absorber in the front.

2.11	The Brakes shall be dual circuit airbrakes, ‘S’ cam type with parking brakes acting on rear wheels.

2.12	Fuel Tank - Capacity shall be minimum 300 Liters with lockable fuel cap.

2.13	The Chassis shall be provided with appropriate Radial Tyres, with two Spare Tyres.

2.14 	The  Chassis  shall  be  provided  with  single  day  type  cab  with  RED  colour  made  from high strength steel fully trimmed, external panels hot dip galvanized with hydraulic cab tilting mechanism. The Cab suspension shall be provided with coil spring and shock absorber. The cab shall be provided with adequate ventilation, rear  view  mirrors,  Wind  Screen  and  Windows,  adjustable  driver  seat,  Wiper system and along with all other standard fitments.

2.15	The  Electrical  system  shall  be	24  Volts  with  suitable  capacity  batteries  & Alternator for charging the batteries.

2.16	The  Chassis  shall  be  supplied  with  standard  tool  kit,  hydraulic  jack  of  20  Ton capacity, Operator & workshop manuals.

2.17 	The Chassis shall be fitted with gearbox mounted suitable capacity Power Take off Unit to drive the Hydraulic pump for boom movement.

2.18 	The  Chassis  shall  be  directly  procured  by  the  tenderer  confirming  to  above specifications  and  shall  be  got  homologated  with  the  appropriate  authority  in India.   The   Transportation   responsibility   of   the   Chassis   up to   tenderers manufacturing facility lies with the tenderer. The Chassis shall be insured while in transit.

2.19   The Chassis shall comply with all the provisions and enactment of Motor Vehicle Act 1988 and Central Motor Vehicle Rules, 1989 and also comply with                Tamil Nadu Government Motor Vehicle Rules and any amendments made from time to time.

3. MAIN OPERATING DATA:

Max. working height					-    Not less than   54.0 Meters

Max. height to working cage bottom  		-    Not less than   52.0Meters

Max. height to the top of the ladder		-    Not less than   53.0 Meters

Max. working outreach at 400 kg cage load, at least- 	                    21.5    Meters

Max. working reach below the ground level 	-    Not less than     5.0 Meters

Safe working load

(without water discharge, minimum of)              -   A minimum of     400 Kg

Safe working load with monitor

(with water discharge) up to 3800 l/pm              -   A minimum of   250 Kg

Rotation, continuous	-	360°

Transport height (depending on chassis) 	- shall not exceed 4.15 m

Transport length (depending on chassis) 	- shall not exceed	12.75 m

Transport width – shall not be more than           - 2.5 Meters

(VOLVO make having a minimum of 400 HP with appropriate wheel base and comply with Euro VI Engine). Lesser time to reach maximum height is preferred.

GVW :                                                                - In the range of 32  – 38 tons

4. MAIN FRAME:

4.1 The  main  load  bearing  element  of  the  aerial  device  shall  be  the  strong  main  frame, which takes all the loads caused by the operation of the aerial.

4.2 The main frame shall be fixed on to the chassis frame with bolts in such a way that chassis performance and durability are maintained.  The  front  fixing  bolts shall be fitted with springs to allow the chassis frame beams to flex when the outriggers are fully down, thus avoiding any stress concentration in the chassis beams. The actual main frame shall be fully welded rectangular steel structure 
providing high stiffness and thus maximum comfort-ability and operational safety.                   At each end of the main frame there shall be integrated housings for outriggers.

5. STABILIZING SYSTEM:

5.1. The  stabilizing  system  shall  consist  of  four  hydraulically  powered  outriggers  mounted in their housings in the main frame. Each housing shall be fitted with  adjustable  guides  to  provide  smooth  and  accurate  movement  of  the  outrigger beam. The horizontal beam shall be a completely closed steel profile enclosing the hydraulic cylinder for the horizontal movement and the hydraulic hoses for the cylinder of the vertical movement thus completely protecting those devices from external damage.

5.2. 	The H-type outriggers shall be chosen for their ability to stabilize the vehicle from behind obstacles and to be placed on raised structures as necessary. Each vertical jack shall be fitted with self-aligning foot plate to spread the load evenly and to allow operation on uneven ground.

5.3. 	The vehicle must be fitted with automatically operating leveling system that is capable  of  performing  the  entire  jacking-up  and  leveling  procedure  within  30 seconds. A fully manual back-up operation must be provided. As standard feature the stabilizing system shall be able to be used in "one sided" mode requiring only whether the left or the right hand side outriggers to be fully extended horizontally where  as  the  outriggers  on  the  opposite  side  remain  within  the  width  of  the vehicle. This feature substantially reduces the width required for setting up and operating the aerial yet it provides full working height and full working outreach when operated on the fully supported side of the vehicle.  There  shall  be  two independent  automatically  operating  and  self  controlling  safety  systems  to prevent an unsafe configuration to be reached when the "one sided mode" is in use. The stabilizing system shall also include a rear axle locking system in such cases where it is required for meeting the stability criteria. All controls for the entire stabilizing system shall be located in dust and water proof locker at the rear of the vehicle. The control levers shall be arranged in such a way that the outriggers are always visible to the operator when operating each side separately. All control levers shall permit infinitely variable speed of each function separately or  all  together  for  easy  and  fast  stabilizing  procedure,  also  when  operated  on uneven ground.

5.4.	In  the  middle  of  the  control  panel  there  shall  be  following  additional  control devices:

− starting of chassis engine

− stopping of chassis engine

− activating hydraulic pressure

− visual indication for fully extended left outriggers

− visual indication for fully extended right outriggers

− visual indication for applied rear axle locking (if the feature is installed)

− visual indication for main current being switched on

− operating hour gauge

− indications for the fault finding system

− switch for the battery driven back-up for the hydraulic system

− visual indicators for leveling of the vehicle (longitudinal and transversal)

5.5. The locker containing outrigger controls shall be fitted with automatically operating door switch and light for night operation.

6. BODYWORK AND EQUIPMENT LOCKERS:

6.1.The frame for the bodywork shall be made of welded steel profile sections bolted together for easy maintenance if required. All parts of the frame shall be hot dip galvanized for long life. The frame shall be covered by non-slip aluminum plate strong enough to allow free movement of persons on it.

6.2. 	Equipment lockers shall be welded together of stainless steel plates, primed and painted  and  finally  bolted  onto  the  frame  for  easy  removal  if  necessary.  All lockers shall be fitted with hinged doors, properly sealed to be water and dust proof.  All  doors  shall  be  fitted  with  automatic  switches  activating  the  lights  as soon  as  the  door  is  opened  and  also  activating  the  warning  in  drivers  cab  to indicate that all doors are not fully closed.

6.3.	To  provide  easy  access  from  the  ground  level  there  shall  be  steps  covered  by non-slip aluminum at each side of the vehicle. At the end of the steps there shall be railings made of stainless steel to assist when climbing.

6.4 A locker of the appropriate size shall be provided at the appropriate place                to house a suitable portable pump.

7. BOOMS:

7.1.	The vehicle shall perform the following functions.

a.	Elevation.

b.	Depression

c.	Extension & housing of telescopic sections, and

d.	Rotation 360 degrees in either direction.

7.2. 	There  shall  be at least two  booms,  the  first  boom  with  telescopic  extension  providing direct movement and angular movement of not less than  83 degrees, and the remaining booms with vertical movement of approx. 180 degrees. This configuration shall have to result in compact travelling dimensions yet in extreme versatility in operation. The second boom shall provide an up-and-over capability of approx. 7 m throughout its vertical movement.

7.3. 	The  main  boom  elevation  and  lowering  shall  be  controlled  by  two  hydraulic cylinders that both have their separate safety devices and both can alone carry the entire load. Tail swing shall not exceed the measurements of the truck in any direction.

7.4. 	The booms shall be welded by the unique plasma welding method to provide high durability and extreme accuracy. For high strength and minimum flexing of the boom  sections  only  high  tensile  strength  steel  shall  be  used  as  load  bearing structure.

7.5. 	All telescopic sections of the first boom shall move synchronized i.e. there shall be no intermediate jerks when the extension / retracting is operated. All sections shall be fitted with adjustable guides to provide smooth and accurate movement.

7.6. Different  maintenance  objects  shall  be  located  well  at  hand  either  outside  the boom or behind easily removable covers.

7.7. All booms shall be internally and externally primed and painted for long life span, treated against rust and corrosion.

7.8. Preferably, all necessary hydraulic, electric, air lines etc up to the cage shall be installed in such a way so that they are well protected from any external damage.

8. TURNTABLE:

8.1.	The  turntable  shall  be  fully  integrated  steel  structure  welded  by  computer controlled  robots  for  extreme  accuracy  and  durability.  The  centre  post  shall contain slip rings with double pins for electrical connections,  100 mm stainless steel  water  way  and  hydraulic  pressure  and  tank  lines  shall  allow  continuous rotation of the turntable.

8.2. 	The  centre  post  shall  be  mounted  inside  of  the  turntable  in  such  a  way  that maintenance  can  be  done  directly  from  the  turntable.  The turntable tail swing shall not exceed the vehicle dimensions in any direction.

8.3. 	Rotation reduction gear with automatically operating braking system shall be installed at the front of the turntable for easy maintenance and adjustment. The hydraulic motor powering the rotation movement shall be fitted directly into the gear for high reliability. At the left hand side of the turntable there shall be the lower control station and at the right hand side the rescue ladder.

9. WORKING CAGE:

9.1. The working cage shall be fixed to the booms with pivoting point at the level of the railing to provide highest possible degree of natural safety. The cage shall be made of tubular steel profile, welded together and painted with special paint with high durability.  The  dimensions  of  the  working  cage  shall  be  1.0  m  (length)  x 2.0 m  (width) x  1.1 m  (height) and three inward opening doors located at suitable places to enable safe access to the cage is a must. Safe working load shall be 400 Kg when no water is discharged.

9.2  2 no of Hardened plastic foldable seats with locking type safety belt shall be provided in the Hand rails of the cage to hold the victim themselves. So that they may feel secure in the time of rescue from high altitudes. Also it will restrict the unwanted panic movement of occupants inside the cage, thus balancing of cage will not be disturbed.

9.3 Cage door locks shall be provided with hard enough locking security with double check lock provision and as well as it shall be easy to operate.

9.4 Water bottle holders shall be provided in the cage.

10.WORKING CAGE LEVELING:

10.1. The working cage shall be kept horizontally leveled in any position of the booms. The  leveling  system  shall  be  controlled  by  an  automatic  horizontal  monitoring device  with  two  fully  automatic  and  independent  safety  circuits  in  case  of  an uncontrolled  leveling  failure.  There  shall  be  a  master  switch  for  the  automatic leveling  system,  thus  it  can  be  isolated  and  the  manually  controlled  system activated.

10.2. The  leveling  movement  shall  be  powered  by  a  hydraulic  cylinder  located  well protected inside the second boom. The cylinder shall be connected to a double mechanical linkage of which each alone can carry the entire safe working load.

11.	WORKING CAGE SLEWING:

11.1. The working cage shall be turned 45 degrees to each side from its centre position. The  movement  shall  be  powered  by  a  hydraulic  cylinder  with  controls  in  the working cage and at the turntable control panels. The centre position of the cage shall be indicated by a visual indication at both control panels.

12. 	FOLD DOWN RESCUE PLATFORM:

12.1. At  the  front  of  the  working  cage  there  shall  be  a  rescue  Platforms  with automatically  operating  safety  railing  to  provide  additional  safety  during  rescue and fire fighting. The dimensions of the rescue Platforms shall be 1.45 m x 0.45 m with minimum of 180 kg load carrying capacity.

13.	RESCUE LADDER:

13.1. A stable telescopic rescue ladder system shall be attached onto the right hand	side of the booms.  Due  to  the  telescopic  design  the  ladder  forms  a  direct continuous  rescue  way  with  no  cross-over  Platforms  or  similar  obstacle.                      The ladder shall be attached onto the boom structure at several points throughout its length  resulting  in  extreme  stability  even  when  operated  in  windy  conditions. Extension movement of the ladder shall be automatically synchronized with the telescopic movement of the first boom requiring no separate control devices.

13.2.  Both  control  panels  shall  be  fitted  with  visual  indication  for  "safe  to  climb"	position  of  the  ladder.  There  shall  be  a  step  at  the  turntable  to  provide  safe	access  from  the  ladder  down  to  the  decking.  Thus, a continuous way from the	maximum height down to the ground shall be provided for high capacity rescue operations. A ground ladder can be attached if so required.

13.3. Telescopic  rescue  ladder  must  meet  norm  EN  1777  clause  3.21  and  clause 5.1.4.1.3  for  rescue  ladder.  Test certificate from the manufacturer shall be submitted with the tender documents.

14.HYDRAULIC SYSTEM:

14.1. Hydraulic power shall be provided by one double axial piston pump, which shall be driven by the vehicle power take-off. The power need may be about 30...50 KW depending on the size of the unit.

14.2. Without any operation of the aerial device, the pump shall rotate on minimum flow and minimum pressure. When one of the movements is operated the control valve shall automatically increases the pressure to a pre-set constant level and the oil flow to the amount that is needed for the movements activated.

14.3.	The loss of power in the hydraulic system, which normally causes overheating of the  hydraulic  oil,  shall  be  avoided  and  also  the  stresses  caused  to  the  vehicle transmission  and  P.T.O.  system shall be minimized.  At the same time fuel, consumption and exhaust emissions shall be kept at the minimum.

14.4.	By  operating  several  movements  simultaneously  the  oil  flow  shall  increase automatically  according  to  the  need  in  the  system  thus  making  all  movement speeds independent on each other.

14.5.	The   constant   pressure   system   with   max.   pressure   setting   shall   prevent Over loading of the system and its components e.g. cylinders.

14.6. Inside of the turntable and at the lower valve compartment there shall be instant Couplings for the manometer in each pressure line. The manometer shall be fitted as standard equipment.

14.7. The filtration of the oil shall consist of suction strainer in the suction line, pressure filters with visual indicators in each pressure circuit, return filter in return line and air  filter  on  the  reservoir  thus  providing  maximum  reliability  by  protecting  the hydraulics against foreign particles.

14.8. The  hydraulic  cylinders  shall  be  double  acting  with  hard  chrome-plated  piston rods and they shall be fastened by means of self-aligning ball bearings to prevent lateral forces from damaging the seals or piston rods of the cylinders.

14.9. Hydraulic oil tank shall be integrated into the main frame for good protection and transformation of the heat. The tank is fitted with oil level gauge, temperature gauge, suction connections with closing valves for easy maintenance and draining outlet with closing valve.

15. 	BACK-UP FOR THE HYDRAULIC SYSTEM:

15.1. There shall be a separate battery driven and petrol driven hydraulic pump which shall  provide  an  independent  means  of  power  in  case  of  failure  of  the  main engine.  The  system  shall  be  able  to  be  started  from  all  control  panels  thus providing an immediate back-up in a case of a failure at an intense fire or similar immediate emergency.

16.	ELECTRIC SYSTEM:

16.1. The electric supply shall be taken from the chassis battery, which is kept, charged when the engine is running.  Voltage  of  the  system  shall  be  24  V  DC  and  all circuits shall be fitted with their specific fuses. When the main current is switched on, yellow flashing warning lights located at each outrigger boom pivoting point and underneath of the working cage shall be automatically switched on.

17. 	SIREN AND PUBLIC ADDRESS SYSTEM:

17.1. There shall be electric siren unit fitted on the roof of vehicle cab. Control panel of the  system  shall  be  conveniently  located  for  the  driver  and  it  shall  includes switches for fast  (yelp) and slow  (wail) sounds. When both switches are on, the system shall automatically change the sound after three periods of each fast and slow.

17.2. 	Command  microphone  which  shall  be  fitted  with  push-to-talk  switch  allow  the public address  message  to  override  the  siren  function.  Operations shall be controlled by lighted push buttons of different colour.

18.	ROTATING BEACONS:

18.1.	On each side of the drivers cab roof there shall be rotating beacons in the Red colour. The switch for switching the beacons on and off with suitable signal light shall be fitted inside of the cab in a convenient position for the driver.

18.2   4 no of blue-red combination flickering LED lights shall be fitted in the front side of the vehicle. Two shall be fitted below the front windshield and the other two shall be fitted on the front bumper..

18.3   2 no big square red blue combination Flickering LED lights shall be fixed in the rear side of vehicle.

18.4  3  no Blue-red  combination flickering LED lights shall be fitted on each side platform of the vehicle.

19.	INTER COMMUNICATION SYSTEM:

19.1.	There  shall  be  fully  transistorized  talk  back  inter  communication  system  fitted between turntable and the cage.

19.2. The  system  shall  be  combined  microphone  and  loudspeaker  for  hands  free operation and shall be located in the cage. The turntable control station shall also be equipped with microphone, which shall be integrated in the loudspeaker with volume control.

19.3. The  microphone  and  the  loudspeaker  shall  be  sealed  properly  and                 it  shall  be protected from the ingress of water, dust and humidity.

19.4 In addition to that One to one communication device shall be provided between cage and turntable operators. It shall be a hands free model and it shall be fixed in the helmet.  So that they can communicate easily between themselves without using their hands when their hands are engaging with joystick operation

20. 	CONTROL SYSTEM FOR BOOM AND ROTATION MOVEMENTS:

20.1. All boom and rotation movements shall be controlled electro-hydraulically by means of servo valves, as because of the servo principle the control function is not sensitive to changes of ambient or oil temperature, thus providing smooth, safe and very accurate movements even in most severe operating conditions. All control movements shall be performed by the remote control system from both control panels.

21. 	TURNTABLE AND WORKING CAGE CONTROL PANELS:

21.1. 	The  turntable  control  panel  incorporating  all  control  levers  and  safety  system indications shall be fitted with a rotatable arm at the side of the turntable. This feature shall enable the whole control panel to be placed and locked conveniently in its operating position to provide the officer in charge with an excellent view over the different indications of the safety systems. If desired, the control panel shall able to be rotated and locked in a position enabling direct access from the decking of the vehicle into control station.

21.2. The control station shall be fitted with convenient seat to provide comfort even in case of prolonged operation. The Platforms underneath the control position shall be covered by non-slip aluminum plate.

21.3  Safety harness provision shall be given to the turn table operator with fast detaching facility. So that the operator can secure himself to avoid falling down from the chassis as well as he can release himself quickly in case of emergency. Safety certification for safety harness shall be supplied to the department along with vehicle.

21.4. The working cage control panel incorporating all control levers and safety system indications shall be fitted with an easily removable bracket to enable the panel to be placed whether at the front of the cage or at the side of it as required by the particular operational task. This feature shall considerably increase the versatility of the unit in actual operation.

21.5. Both control panels shall be exactly alike thus considerably reducing the risk of confusion amongst operators under stress or even panic.

21.6.	At the turntable control panel there shall be a change-over switch to select the panel from which the operation is controlled.

21.7. Both  control  panels  shall  be  fitted  with  following  most  important  warning, indication and control devices, all marked by clear symbols for easy recognizance.

−   visual and audible indication for exceeding safe working load

−   visual warning for activation of working cage collision guard system

−   visual indication for fully extended left outriggers

−   visual indication for fully extended right outriggers

−   visual indication for the rescue ladder "safe to climb"

−   visual indication for the centre position of the booms

−   visual indication for the centre position of the working cage

−   starting and stopping of chassis engine

−   switch for the battery driven back-up for the hydraulic system

−   separate  joystick  control  levers  for  each  movement  pair  (first  boom up/down,  telescope  sections  in/out,  cage  boom  up/down,  turntable rotation left/right)

−   control lever for cage slewing

−   emergency stop button

−   overriding of the automatic working cage leveling system

−   manual operation for the working cage leveling system

−   switches for activating the bleed down system

22. 	CONTROLS AND INDICATORS IN DRIVERS CAB:

22.1.	In addition to chassis standard controls and indicators the following items shall be installed in drivers cab:

−   visual warning for the main current being switched on

−   visual warning for any of the equipment lockers being open

−   visual warning for the booms not being fully in traveling position

−   visual warning for the rear axle being locked (if the feature is installed)

−   visual warning for the outriggers not being fully in traveling position

− switch with visual indication for rotating beacons (optional)

−   switch with visual indication for siren unit

−   microphone for the public address system

23.	SAFETY DEVICES:

23.1. All  load  bearing  hydraulic  cylinders  shall  be  fitted  with  lock  valves  directly integrated into the cylinder structure to prevent the booms, the working cage or the outriggers from retracting in case of a pipe or hose failure.

23.2. Retracting of any of the outriggers shall be automatically prevented as soon as the booms have been lifted from their traveling position.

23.3. Similarly lifting of the booms from the traveling position shall be prevented until the outriggers have reached the support width and ground pressure.

23.4. All boom movements shall be limited at their most extreme positions thus making it impossible for the operator to reach an unsafe configuration by normal means of operation. The movements having direct influence on the stability of the aerial shall  be  fitted  with  two  separate  limiting  circuits,  the  first  one  stopping  that particular  movement,  the  second  one  deactivating  the  whole  electric  and hydraulic system should the first circuit not have worked.

23.5. The major movements, lifting of the first boom to its maximum elevation, and extending the telescopic movement or lowering the first boom at the maximum outreach shall be fitted with slow-down devices to provide smooth deceleration of the movement.

23.6. Starting of the chassis engine from any of the control panels of the aerial shall be prevented unless the gear is shifted to neutral.

23.7. Inadvertent damaging of the drivers cab by the first boom shall be prevented by a system preventing lowering of the first boom and rotation movement when the first boom is near the drivers cab.

23.8.	An overload warning shall be fitted to give an audible and visual warning in case of exceeding the safe working load.

23.9. A  collision  guard  shall  be  fitted  to  provide  additional  safety  when  operating  in darkness  or  in  dense  smoke.  This system shall stop all movements and gives visual warning when activated.

23.10. An  emergency  stop  switch  shall  be  fitted  at  both  boom  control  panels  to provide  immediate  and  complete  "freezing"  of  all  systems  in  case  of  an unexpected emergency.

23.11. The  control  system  shall  be  fitted  with  RPM  switches  to  provide  additional safety.

23.12. There shall be a "bleed down" system which can be operated from working cage and turntable control panels. By means of this system the booms can be lowered  and  the  working  cage  brought  down  onto  the  ground  even  if  no hydraulic pressure is available. In such a case manual rotation is provided by manual means.

23.13. The  limiting  system  of  the  aerial  ladder  Platforms  must  meet  safety  norm EN  954-1 and electromagnetic compatibility must be certified by EMC test certification. The certification for both must be submitted along with tender documents.

23.14. Fault Finding System: The control system of the vehicle shall have self fault finding system. If any fault occurs during the operation the system shall find out the same and shall show the location of the defective component on the display.

24.	WATER WAY SYSTEM:

24.1. The water way system shall be completely made of stainless steel. The nominal diameter of the water way is 100 mm. There shall be one 2½" (68 MM) male inlet with a closing valve at each side at the rear of the vehicle from where the line leads through the centre post in the turntable up into the working cage where the water monitor is mounted. Along the booms, the piping is fitted onto the right hand side in a well protected position between the boom and the rescue ladder.

24.2. The  centre  post  shall  be  mounted  in  the  centre  line  of  the  turntable,  provides continuous rotation even if water supply is simultaneously used.

24.3.	The piping shall be protected from possible overpressure by means of two relief valves mounted underneath of the turntable.

24.4. On  the  side  of  the  first  boom  there  shall  be  a  telescopic  water  pipe,  made  of stainless  steel.  Moving sections of this pipe shall be externally ground and chromium plated to provide reliable function and long life span.

24.5. Seals  between  the  sections  shall  be  of  low  friction  type  and  shall  be  easily tightened if so required. On the side of the second boom a fixed stainless steel pipe shall be installed and in boom pivoting points flexible, specially reinforced 100 mm pressure hose is used. All hoses shall be fixed to the pipe with reliable span-lock connections.

24.6. Piping shall end at the right hand side at the front of the working cage where the water monitor is placed. A 75 mm valve shall be fitted in the cage to isolate the monitor if required.

24.7. There shall be an additional outlet with 63 mm (2½") closing valve and coupling for water supply from the cage through an extension hose.

24.8.	All Fire Hose couplings shall be 63 MM (2 ½”) diameter instantaneous male and female delivery hose couplings.

24.9.There shall be drain cocks fitted in the piping to enable it to be drained after use.

24.10. On the front underneath of the cage there shall be nozzles of water spray curtain system to protect cage occupants from radiant heat. Control valve of water spraycurtain system shall be conveniently located inside of the cage

25. WATER MONITOR:

25.1. Water monitor shall be connected onto the piping system and it shall be placed at the front of the cage on the right hand side just outside of the railing to utilize to the entire cage floor area in extreme rescue situations. The monitor shall have  maximum nominal capacity of 3800 LPM with no friction loss at the monitor with adequate supply pressure and having a provision of Remote Control.

26.	PAINTING:

26.1.  Before  painting  all  surfaces  of  steel  structures  shall  be  carefully  shoot  blasted	after which they shall be primed. After the final top paint the thickness of the	paint film shall be 100 microns.  All booms shall be painted up to the same specifications from inside.

26.2. To provide very high corrosion resistance hollow structures such as steel profiles of the working cage, working cage support, cage boom and outrigger beams and housings shall be treated with anti-corrosion protection "TECTYL".

26.3. The words “Tamil Nadu Fire and Rescue Services” shall be painted on both sides of	the    vehicle in English and Tamil at suitable places in consultation with the Department.

26.4.   The emblem of Tamil Nadu Fire and Rescue Services shall be painted on both sides of the vehicles in Natural colour.

26.5. Paint tones used for standard units shall be:

Working cage     -	white aluminum -     RAL 9006 Working cage support, boom sections, turntable and related cylinders – white - RAL 9010 Mainframe, outriggers and bodywork red    -red	-RAL 3000 Chassis frame touch-ups - chassis original tone

27.ACCESSORIES:

4 pc   Wooden outrigger ground pads with brackets

2 pc   Working range diagrams, one at the turntable, one in the working cage 1 pc   Marking of safe working load in the working cage

2 pc   Unit type marked at the boom

5 sets of Warning labels and instruction plates 5 sets of Operation and maintenance manuals

1 pc   Plug for 24 V working light at the turntable and in the cage 1 pc 24 V  70 W working light with universal bracket

1 pc   Lifting loop under the working cage, capacity 400 kg

4 sets Fitment for safety belts in the working cage

1 pc   Hydraulic pressure gauge

4 pc    Locking pins for the outriggers

1 pc    Battery operated Public Address system.

1 No.   Battery operated Hydraulic Combi-Tool with battery charger and spare battery.

1 pc remote control for water monitor.

Wind speed meter showing the wind speed both of the cage and turntable.

4 Nos. of Personal protective Suits (Multi layered) as per the following norms.

Norms for personal protective suit: The suit shall be of three layers, in two pieces of pant and coat/jacket. Three layer personal protective suits shall confirm to norms as specified in EN 469, protective clothing for firefighters and a certificate to this effect from the reputed institute of testing shall be sent along with the offer. The colour of the pant and coat shall be of blue and retro – reflective tape of 50 mm wide and also with an insignia of ‘T.N. Fire and Rescue Services’ shall be provided on specific portion of Jacket. Reflective tapes have to be provided around the body and sleeves. The gloves confirming to EN 659, boots confirming to EN 345, helmet with torchlight confirming to EN 443 and hood as per norms and test certificates to this effect from the reputed institute of testing shall be sent along with the offer. The suit and accessories shall be certified to mentioned EN (or equivalent) standards.

A breathing air system shall be provided from turntable to working cage. At the cage there shall be a 4-way manifold with instantaneous couplings to connect 4 breathing masks. Air cylinders to supply the breathing air shall be mounted at the turntable. The cylinder capacity shall be such that they provide minimum 7000 L of free air. Isolation valve shall be provided at suitable location so that the cylinder can be changed without interrupting the air supply. Pressure regulator as required shall also be incorporated in the system. Suitable face mask with adequate length hose for breathing (4 nos.) shall be supplied. The breathing mask hose shall have enough length to allow the users to roam inside the cage without any restriction.

Vehicle Miniature:Table top operational miniature shall be supplied along with vehicle. It shall be an operational model with separate joystick control to give training to the crew members before allowing them to operate the vehicle. The miniature shall be as similar to the vehicle model proposed to be supplied to the department.

Further Spares required for the maintenance of the Aerial ladder platforms and to be supplied with the following items:

For Chassis:

- Engine oil filters 2 sets (2) Diesel filters  - 2 sets (3) Air filters-1 set (4) Fan belts-1set

(5) Fuses - 1 set and (6) Accessory tools-  1 set each

For sky lift platforms:

(1) Hydraulic filters – 1 set  (2) Fuses-1 set (3) Fault finding guides

(4) Suitable spare sensor-1 set

--------------

QUALIFICATION CRITERIA

- The orders issued under Rule no.144 (xi) of General Financial Rules   (GFRs) 2017 relating to restrictions on procurement from a bidder of a Country which shares a land border with India will be applicable for this tender as per the guidelines issued by the Government of Tamil Nadu vide G.O.Ms.no.343 Finance (Salaries) Department Dated 18.09.2020. The Bidder has to undertake compliance of this and any false declaration and non-compliance of this would be a ground for rejection of their tender.

|  | The manufacturer should be ISO 9001 certified company for the marketing, design, and manufacturing and after sales service of aerial platforms for fire fighting and rescue operations. Further, the manufacturer should have ISO 14001certificate for their manufacturing plants. |
| --- | --- |
|  | 3rd party certificate from an independent agency for welding quality certified to ISO 3834-2:2005 for load bearing steel structures for mobile hydraulic aerial appliances or other internationally reputed certificates for welding quality acceptable to the departmental technical committee should be produced. |
|  | 3rd party certificate from an independent agency for electromagnetic compatibility for the electronic (computerized) control system, for electromagnetic compatibility of wireless remote control system should be produced. Further, 3rd party certificate as per EN 954-1 certifying the safety of the limiting system as per the machinery directives should be produced. |
|  | EC type certification (CE certificate as per EN 1777), 3rd party notified for the quoted model |
|  | The manufacturer must have trained manpower in India preferably in Chennai either directly or through its authorized after sales service partner to take care of maintenance and after sales service of the platform and should be able to enter into an comprehensive maintenance contract with the Tamil Nadu Fire and Rescue Services Department. |
|  | A performance certificate shall be submitted along with the tender from a fire brigade, preferably from India along with copy of orders received. |
|  | The tender should be submitted by the manufacturer or their authorized agent in India. |
|  | Manufacturer shall have minimum 15 years of experience & shall have manufactured & supplied minimum 100 nos. of Hydraulic Platforms/ Aerial Ladder platforms all over the world in the last 5 years including 20 nos. of similar or higher height. |
|  | Manufacturer shall have supplied minimum 5 nos. of hydraulic platforms for fire and rescue purposes in India of same or higher height in the last 5 years. Purchase order copies to be enclosed. Performance certificates of at least 2 such vehicles to be submitted with the offer. |
|  | Certificate from International power access federation (IPAF) certifying the OEM as an approved training center for Aerial Platforms and to show the quality of trained instructors for providing quality training on these appliances. |

Criteria for Evaluation of Tenders

- This is a two-bid tender  consisting of:

(i) technical  bid

and

(ii) price bid

- Technical bid which may be named as “Envelope-A” should contain the technical details of the items offered, commercial terms and conditions along with a list of the enclosures such as  proof in respect of manufacturer or the authorization letter from the manufacturer, latest local tax clearance certificate if any applicable to the country/origin along with EMD

- Price bid, which should be furnished separately as “Envelope-B” in the format prescribed in the tender form in a sealed cover, and the same will be opened only after evaluation of the technical bid

- The technical bid evaluation of the tender offers will be done in three stages. The rules laid down in Tamil Nadu Transparency of Tenders Act, 1998 and related rules, as amended from time to time, will be scrupulously followed.

- If at any stage, a technical flaw is noticed in following the procedures, the procuring entity will take appropriate decisions and reserve the rights to relax or waive or modify at his discretion any of the condition in the interest of the Department so as to ensure that the overall objective of procurement is not defeated.

Stage I: Rejection of tender offers found technically defective:

- Only the tender document supplied by this Office or the tender documents which have downloaded from the department’s website free of cost should be used and it should be resubmitted in full shape without detaching any page from it.

b) Each page of the tender document, including technical specifications should  be duly signed for having noted the contents of the tender document. the tenderers are requested to put the page number to the tender  documents serially including all enclosures while submitting their tender.

- Tenders in any forms other than the forms and schedules issued by the department to the tenderer will not be considered and will be rejected as              non-responsive.

(i) Those who do not submit the EMD, as defined in this tender document,  along with the tender (unless the vendor is exempted by a specific rule provision for which documents must be submitted along with the tender to prove that he or she is exempted for this particular item) will not be considered.

(ii) In case of partial amount of EMD, the tender will be rejected straightaway and the offer will not be considered. In cases where exemption is sought, a plea for producing the exemption documents later will not be accepted.

(iii)  EMD submitted in a manner other than that specified in the tender document  will be considered as not submitted and the tender will be rejected.

- (i) A vendor who does not answer all the questions in the Questionnaire will             not be considered. Only those offers will be considered in which all questions in the Questionnaire have been answered.

(ii) Oversight will not be taken as a reason for not answering a question. It is presumed that a vendor trying for a deal about equipment related with life-saving would show adequate seriousness about furnishing details about the product offered by him or her.

- While supplying information in response to the Questionnaire answers such as “will be supplied later” or “will be intimated later” will not be accepted.  Such answers will lead to the conclusion that the vendor is either not an authorized dealer of the said company or is not serious about the deal.

Stage 2.(Evaluation of Essential Features as per specifications in the tender form)

The offers that are not rejected at Stage-1,  will be examined for the essential features noted below. The information for such evaluation will be taken from those pages of the answers to the questionnaire, which have been signed and carry the seal of the vendor.

(Kindly note that information on the pages of the answers to the questionnaire, which are not signed, or do not carry the seal will be ignored. The information on any pamphlets or brochures submitted by the vendor may be used for corroboration and cross checking of the information supplied by him or her in answer to the questionnaire but may not be used unless this information is provided in a signed answer-sheet carrying the seal. An answer like “Please see the marked portion of the pamphlet or brochure attached” may be taken as not answered. The whole transaction will be conducted on the presumption that a serious vendor knows what he or she is selling)

The following essential features will be considered to qualify at this stage for further evaluation.  Failure to comply with the following essential features, the tender will be rejected

- The tenderer should fulfill the qualification criteria prescribed in this tender.

- The manufacturer should attach a copy of proof of approvals given by the appropriate authority to satisfy all the qualification criteria mentioned in the specification part, failing which the tender will be rejected.

- The manufacturer must have trained manpower in India preferably in Chennai either directly or through its authorized after sales service partner to take care of maintenance and after sales service of the platform and should be able to enter into an comprehensive maintenance contract with the Tamil Nadu Fire and Rescue Services Department.

- A performance certificate shall be submitted along with the tender from a fire brigade, preferably from India.

- The tender should be submitted by the manufacturer or their authorized agent/distributor  in India.  Attach a copy of  proof for the same

- Certificate of approval of the standard by the competent authority in respect of the product offered . Attach a copy of proof for the same

- The answers given by the tenderer to the Questionnaire will be taken into account for preliminary verification whether the offer made by the tenderer satisfy the specification and other requirements.

Failure to comply with the above instructions and an offer which does not meet out the detailed specifications will be rejected.

.

(However, the procuring entity will take appropriate decision and reserve the rights to relax or waive or modify at his discretion any of the condition / specification in the interest of the Department so as to ensure that the overall objective of procurement is not defeated.)

Stage 3: Selection of the suitable tenders:

- The offers that are not rejected during stage 2 above will be further evaluated for selection of the suitable items.

- Since the tender is for fabrication and supply of “Aerial Ladder Platform” as per specifications given in the tender document, further evaluation will be made based on the detailed drawing of the body, water tank, pump and PTO mounting, to be submitted along with tender and also with reference to the details given to the questionnaires in the tender document.

- While evaluating the tender, any additional details may be called for from the tenderers as per the discretion of the DFRS for which a message will be dispatched to him through Fax. Hence, it is the responsibility of the tenderer to furnish the additional particulars as and when called for.   Furnishing of such additional details will, however, be no indication that there is going to be accepted.  In case, any vendor fails to furnish the details called for within the stipulated time, it will be presumed that he is unable to furnish the details and not interested in his offer and his offer will be rejected.

- While evaluating the technical bids If the offered models of the tendered Aerial Ladder Platform does not suit our conditions as laid down in technical specifications, the Director of Fire and Rescue Services reserves the right:

- to relax or waive or modify at his discretion any of the condition / specification in the interest of the Department

(2)  to reject all the Tenders  in respect of this item without assigning any reasons whatsoever and to suitably modify the numbers of Aerial Ladder Platforms to be bought of tendered items of Aerial Ladder Platforms to suit our requirement – consider and suitably modify

Price bid opening

- The Vendors whose offers have been found to be suitable for the use of this department based on the above mentioned criteria for evaluation alone will be called for opening of their price bid at a later date and time fixed by the department which will be intimated to the tenderer after the technical evaluation of the tender is completed.  .

- The Price bid should be furnished in the format given in the tender form in a sealed cover as indicated in the tender for each model separately.  The price bid furnished by the tenderer should contain the break up details of basic price, Custom duty, Octroi, insurance for delivery, freight charges including charges applicable for delivery at  Headquarters, Egmore, Chennai and such other levies. If the price bid not furnished correctly the same will be liable for rejection.

- The Price bid will be evaluated as per the criteria laid down in  Tamil Nadu Transparency in Tender Rules 2000

-0-

Questionnaire to be filled by the Tenderer in respect of tender for supply of                       “Aerial Platform not less than 54 meters”

Note:

- Kindly give to the point and specific answers on a paper or letter head of your Company and put your signatures and seal at the end.  In case you use more than one paper or letterhead, please sign and put seal on each page

- Information given on unsigned papers or letterhead will not be taken into account.

- The vendor if he wishes, may, apart from the answers to the questionnaire enclose brochures and leaflets. However, the information on any pamphlets or brochures submitted by the vendor may be used for corroboration and cross-checking of the information supplied by him or her in answer to the questionnaire but may not be used unless this information is provided in a signed answer-sheet carrying the seal.

- It will be presumed that a vendor who does not answer the questions in this questionnaire is either hiding the information or is not serious about selling his product to the Department.

Questionnaire

(Attach separate sheet or letter head  to furnish the details)

| 1 | What are the name, address and the e-mail address of your Company? |  |  |
| --- | --- | --- | --- |
| 2 | (a) Are you a manufacturer? If so attach a proof for the same or (b)Are you an authorized distributed/dealer of the manufacturer? If so attach a proof for the same 3. Whether you are fulfilling the Qualification Criteria prescribed in this tender document? If so, attach your specific compliance report on the Qualification Criteria. |  |  |
| 4. Describe the following technical aspects in respect of your offered model of Aerial Platform not less than 54 meters. |  |  |  |
| 1. | GENERAL REQUIREMENT |  |  |
|  | Make and Model |  |  |
|  | Standard and Approval |  |  |
|  | Details with reference requirement of General description given in the specification 1.1. to 1.12 |  |  |
| Details to be furnished in respect of their offer with reference to the detailed specifications given in the tender document for each item separately under the following headings |  |  |  |
| 2. | Chassis |  |  |
|  | Make |  |  |
|  | Model |  |  |
|  | Compliance to emission norms |  |  |
|  | Wheel base |  |  |
|  | Engine type |  |  |
|  | Rated HP |  |  |
|  | Type of clutch |  |  |
|  | Type of gear box |  |  |
|  | Type of front axle |  |  |
|  | Type of rear axle |  |  |
|  | Type steering system |  |  |
|  | Type of breking system |  |  |
|  | Fuel tank capacity |  |  |
|  | Size of tyres |  |  |
|  | Type of cabin |  |  |
|  | Type of electrical system |  |  |
|  | Type of PTO |  |  |
| 3 | Main Operating Data |  |  |
| (a) | Maximum working height |  |  |
| (b) | Maximum height to working cage bottom |  |  |
| (c) | Maximum height to the top of the ladder |  |  |
| (d) | Maximum working out reach at 500 Kg cage load |  |  |
| (e) | Maximum working out reach below the ground level |  |  |
| (f) | Safe working load (without water discharge, minimum of) |  |  |
| (g) | Safe working load with monitor (with water discharge) upto 3800 lit. per minute Rotation continuous 360 degree. |  |  |
| (h) | Transport height (depending on chassis) |  |  |
| (i) | Transport length (depending on chassis) |  |  |
| (j) | Transport width |  |  |
| (k) | Typical weight with chassis/ GVW |  |  |
| 4. | Main Frame- 4.1 and 4.2 |  |  |
| 5 | Stabilizing system – 5.1 to 5.5 |  |  |
| 6 | Body work and Equipment Lockers- 6.1 to 6.4 |  |  |
| 7 | Booms |  |  |
| 8 | Turn Table- 8.1 to 8.3 |  |  |
| 9 | Working Cage |  |  |
| 10 | Working Cage Leveling |  |  |
| 11 | Working Cage Slewing |  |  |
| 12 | Fold Down Rescue Platform |  |  |
| 13 | Rescue Ladder in respect of Aerial Platform for 54 mt - 13.1 to 13.3 |  |  |
| 14 | Hydraulic System – 14.1 to 14.9 |  |  |
| 15 | Back-Up for the Hydraulic System |  |  |
| 16 | Electric System |  |  |
| 17 | Siren and Public Address System- 17.1 to 17.2 |  |  |
| 18 | Rotating Beacons & Flickering lights |  |  |
| 19 | Inter Communication System- 19.1 to 19.3 |  |  |
| 20 | Control System for Boom and Rotation Movements |  |  |
| 21 | Turn Table and Working Cage Control Panels- 21.1 to 21.6 |  |  |
| 22 | Controls and Indicators in Drivers Cab |  |  |
| 23 | Safety Devices – 23.1 to 23.14 |  |  |
| 24 | Water Way System- 24.1 to 24.10 |  |  |
| 25 | Water Monitor |  |  |
| 26 | Paintings – 26.1 to 26.5 |  |  |
| 27 | Description of the accessories offered as per specification |  |  |
| 28 | Describe the tools kit/spare parts (make and model, technical details etc) proposed to be supplied |  |  |
| 29 | Whether detailed drawing of the body has been attached with the tender. |  |  |
| 30 | State clearly if any deviations are required from the specifications given in the tender document for each item |  |  |
| 31 | Describe about the provision made in your tender for Inspection, Training and Commissioning |  |  |
| 32 | Warranty period offered for each item |  |  |
| 33 | Whether willing to continue the service of the vehicle upto a period of 20 years from the date of delivery? |  |  |

-0-

IMPORTANT  INSTRUCTIONS TO TENDERERS

1. Source of funds:

The Government of Tamil Nadu have sanctioned money towards the purchase of this vehicle for fire fighting and rescue operation for the use of Tamil Nadu Fire and Rescue Services Department.

2.  Eligible Tenderers

2.1.This Global Tender  for supply of Aerial Platform of no not less than 54 m is invited directly from reputed manufacturers or through their authorized distributors/dealers.

2.2. The orders issued under Rule no.144 (xi) of General Financial Rules   (GFRs) 2017 relating to restrictions on procurement from a bidder of a Country which shares a land border with India will be applicable for this tender as per the guidelines issued by the Government of Tamil Nadu vide G.O.Ms.no.343 Finance (Salaries) Department Dated 18.09.2020.

3  Eligible Goods and Services:

3.1.  All goods and ancillary services to be supplied under the contract shall have their origin.

3.2	For purpose of this clause, “Origin” means the place where the goods are mined, grown or produced or from which the ancillary services are supplied.  Goods are produced when, through manufacturing processing or substantial and major assembling of components a commercially recognized product results that is substantially different in basis, characteristics or in purposes or utility from its components.

3.3.The origin of goods and services is distinct from the nationality of the Bidder.

4.   Cost of Bidding:

4.1. The Bidder shall bear all costs associated with the preparation and submission of its bid, and the Tamil Nadu Fire and Rescue Services Department,  hereinafter referred to as “the Purchaser’ will in no case be responsible or liable for those costs, regardless of the conduct or outcome of the bidding process.

  - The Bidder is expected to read all instructions, forms terms and specifications in the Bidding Documents.  Failure to furnish all information required by the Bidding Documents or submission of a bid substantially responsible to the Bidding Documents in every respect will be at the Bidder’s risk and may result in the rejection of its bid.

5.  Amendment of Bidding Documents:

5.1. At any time prior to the deadline for submission of Bids, the Purchaser may, for any reason, whether at its own initiative or in response to a clarification requested by a prospective Bidder, modify the Bidding Documents by amendments.

5.2 The amendment will be notified in writing to all prospective Bidders, which have received the Bidding Documents and will be binding on them.

  - In order to afford prospective Bidders, reasonable time in which to take the amendment into account in preparing their bids, the purchaser may, at its discretion, extend the deadline for the submission of bids.

#### Language for preparation of bids

Since it is a Global Tender, the Bid prepared by the Bidder from abroad and all correspondence and  documents relating to the bid exchanged by the Bidder and the Purchaser, shall be written in English language, provided that any printed literature furnished by the Bidder may be written in another language so long as accompanied by an  English translation or  Tamil of its pertinent passages in which case, for purposes of interpretation of the bid, the English or Tamil translation shall govern.

# Earnest Money Deposit

7.1. Earnest Money Deposit amount  in Indian Rupee value of  Rs.12,00,000/-            ( Rupees Twelve lakhs only) should be paid along with tender by way of Demand Draft in favour of the Director of Fire & Rescue Services  Department, Chennai should be compulsorily appended with the tender document.  Specified small savings instruments or irrevocable bank guarantee from a Nationalized Bank  will also be accepted.   Personal / Company cheques will not be accepted. If OEM from abroad is directly participating,   EMD may be paid through their representative in India

7.2. Any tender submitted without EMD and partial payment of EMD will be summarily rejected.

7.3. In case of unsuccessful tenderers, their Earnest Money Deposit will be returned to them at their cost in due course soon after the final decision is taken on the tenders by the department.

7.4.  The Earnest Money Deposit will not bear any interest.

8.  Mode of submission of tender offer:

- (a) Sealed tenders containing :

  - Technical bid which may be named as “Envelope-A” containing technical details, commercial terms along with a list of the enclosures such as  proof in respect of manufacturer or the authorization letter from the manufacturer, Questionnaire format duly filed and all Qualification Criteria documents, Product flyer/brouchures, Approval Certificate etc, latest local tax clearance certificate if any applicable to the country/origin along with EMD;

and

(ii)    Separate price bid as “Envelope-B” in a sealed cover in the format prescribe in the tender form  for supply of Aerial Ladder Platform not less than 54 meter

will be received for and on behalf of the Tamil Nadu Fire and Rescue Services Department so as to reach on or before the due date prescribed.                        All the tenders shall be prepared and submitted strictly in accordance with the instructions set forth herein.  The Tamil Nadu Transparency in Tenders Act 1998 and the Tamil Nadu Transparency in Tender Rules 2000 are applicable for the Tender.

(Sealed Tenders means, it may be wax sealed or any other mode of seal at tenderer’s  risk which could not be tampered)

(b) The Earnest Money Deposit in the prescribed format or, if exemption from EMD is sought, proof of exemption from payment of EMD and undertaking in lieu of EMD should be produced along with sealed tenders.

(c )The following address must be written on the OUTER ENVELOPE containing the technical bid cover and two Price bid covers:

Deputy Director –Administration,

O/o. the Director, Fire and Rescue Services Department,

17 Rukmani Lakshmipathy Road, Egmore

Chennai-600008, Tamil Nadu, INDIA.

This cover shall be superscribed “Tender for supply of Aerial Ladder Platform”.  This cover must reach the addressee i.e. Deputy Director – Administration on or before   the due date and time mentioned at the top sheet of this tender document.

(d)  The tenderer has the option of sending the offer by Registered Post A/D or by speed post or by Courier  or by air mail for overseas delivery  before the date and time fixed.  The tenderers who could not send their tender by post can put the tender in the Tender Box kept for this purpose at the above mentioned address before the date and time fixed.

(e) Delivery of tender in person will not be accepted on any account and the tender submitted after the last date and time will not be accepted under any circumstances.

(f) The slit of the tender box shall be 28 cmx 3 cm and therefore, the cover should be brought of suitable size to move inside the slit.

9.  Price:

9.1.The price shall include the cost of Chassis in respect of Aerial Ladder Platform   NOT LESS THAN 54 m

9.2. The price shall include fire fighting equipments and accessories described in the specification.

  - The Bidder shall indicate on the appropriate Price Schedule attached to these documents the unit prices and total Bid Prices of the goods it proposes to supply under the Contract and in case of goods of foreign origin in F.O.B. & C.I.F. cost.

  - Prices indicated on the price Schedule shall be entered separately in the following manner: -

For goods offered from within the Purchaser’s country:

9.4.1.  The price of the goods, quoted ex-factory, ex-showroom, ex-warehouse or off the –shelf,   as applicable including all customs duties and sales and other taxes already paid or   payable;

9.4.2. On the components and raw material used in the manufacture or assembly of goods  quoted ex-factory; or

9.4.3. On the previously imported goods of foreign origin quoted ex-show rooms, ex-warehouse or off-the-shelf.

    - Any purchaser country sales and other taxes which will be payable on the goods if this contract is awarded;

9.4.5.Charges for inland transportation, insurance and other local costs incidental to delivery of the goods to their final destination;

9.6.  The Bidder’s separation of price components in accordance with Para. 7 above will be solely for the purpose of facilitating the comparison of bids by the Purchaser and will not in any way limit the Purchaser’s right to contract on any of terms offered.

- BID CURRENCIES :

10.1.  Price shall be quoted in the following currencies:

- For goods and services which the Bidder will supply from within the Purchaser’s Country, the prices shall be quoted in Indian Rupees: and

- For goods and services which the bidder will supply from outside the Purchaser’s country, the prices shall be quoted either U.S. Dollars or Euro or in the freely convertible currency.

10.2. Further, a Bidder expecting to incur a portion of its expenditures in the performance of the Contract in more than one currency, and wishing to be paid accordingly, shall so indicate in its bid.  In such a case, either (i) the bid shall be expressed in different currencies and the respective amounts in each currency together making up the total price, or (ii) the total bid price shall be expressed in one currency and payments required in other currencies expressed as a percentage of the bid price along with the exchange rate used in such calculation.

11. Conversion to Single Currency:

To facilitate evaluation and comparison, the Purchaser will convert all bid prices expressed in the amounts in various currencies in which bid price is payable; to the Indian Rupees at the selling exchange rate established by the Reserve Bank of India/State Bank of India.

12.	Delivery and Documents:

12.1.  No goods shall be shipped or delivered until an intimation, in writing has been applied for an obtained by the supplier from the Director of Fire and Rescue Services  that the goods may be delivered.

12.2  Before dispatch from the Contractor’s works all goods shall be adequately protected by painting or by other suitable means for the whole period of transit and storage in a tropical humid climate prior to use, against corrosion and incidental damage including the effects of vermin, strong sunlight, rain, high temperature and humid or salty atmosphere or sea spray.  The supplier shall be held responsible for the goods being so protected as to ensure spray.  The supplier shall be held responsible for the goods being so protected as to ensure that as far as is practicable it reaches its ultimate destination intact and undamaged.

12.3. The supplier shall be deemed to have included in the bid prices for all material and packing crates or cases appropriate to the particular item of goods necessary for the safe package, conveyance and delivery of the goods.

12.4. The supplier shall arrange to ensure that each box or unit of shipment is legibly and properly marked for correct identification to supply the item to the purchaser. Failure to comply with this requirement shall render the Contractor liable for any additional expenses involved.

12.5. The Contractor shall give complete shipping information concerning the weight, size and content of each package including any other information the purchaser may require

The following documents shall be airmailed to Purchaser within 7 days from the date of shipment:

- Three copies of the Supplier’s invoice showing Goods description, quantity, unit price, total amount:

(ii)	Original and duplicate copies of the negotiable, clean on-board bill of lading marked freight prepaid and two copies of no-negotiable bill of lading:

  - Three copies of packing list identifying contents of each package:

  - Insurance Certificate in triplicate

  - Manufacturer’s/Supplier’s guarantee certificate in duplicate.

  - Three copies of Inspection certificate, issued by the nominated inspection agency and the Supplier’s factory inspection report :  and

  - Certificate of origin in duplicate

13.	Insurance:

13.1. The Contractor shall provide for insurance coverage for an amount equal to the CIF or  FOR value as appropriate plus an additional 10 per cent thereof and the same shall extend from the date of dispatch of the goods until its arrival at the final designated destination: Chennai Port.  The insurance policy should provide coverage against all risks, including those of wars riot, strikes and malicious damage.

13.2. If the goods or any portion thereof is damaged or lost during transit, the purchaser shall give notice to the Contractor setting forth particulars of such goods damaged or lost during transit.  The replacement of such goods shall be effected by the Contractor within a reasonable time to avoid unnecessary delay in supply of goods. The replacement price of items shall be recovered by the Contractor from the insurance coverage provided for this purpose.   The import license/port clearance, permit, etc. in respect of a foreign supply contract, required for the purpose shall be made available for such goods.  In the case of contracts awarded to domestic contractors, the replacement of goods damaged during transit shall be made free of cost by the Contractor.

14.	Transportation:

The goods are to be delivered at site in India at Head Quarters of Tamil Nadu Fire and Rescue Services Department at no.17, Rukumani Lakshmipathi Salai, Egmore, Chennai-8  from Chennai Port.  Prices shall be inclusive of cost of the goods free on board at the time of shipment plus the cost of marine freight, land freight and insurance of the goods from warehouse to warehouse.

15.	Incidental Services:

As specified in the specifications and in the tender document, the Supplier may be required to provide any or all of the following services;

(a) The manufacturer/ supplier should furnish an undertaking at the time of placing orders to  give warranty for the complete unit including chassis for a period of 5 years from the date of acceptance of the vehicle. Further, all the spares should be supplied at free of cost during the above warranty period. Apart from giving 5 years warranty with replacement of parts during that period, service should be continued up to a period of 20 years from the date of delivery.

(b)  Performance & supervisions of on-site assembly and start-up of the supplied goods

(c)   Furnishing of tools required for assembly and maintenance of the supplied goods.

(d) Furnishing of a detailed operations & maintenance manual for each appropriate unit of the supplied goods.

(e)   Performance, supervision, maintenance and repaid of the supplied Goods, for a period of time agreed by the parties, provided that his service shall not relieve the supplier of any warranty obligations under this contract.

(f)  Conduct, training of the Purchaser’s personnel, at the Supplier’s plant and      on-site, in assembly, start-up, operations, maintenance and repair of the supplied goods as mentioned in the

16. Payment:

16.1. The method and conditions of payment to be made to the Supplier under Contract are specified in the Special Conditions of Contract.

16.2. The Supplier’s request(s) for payment shall be made to the Purchaser in writing, accompanied by an invoice describing, as appropriate, the Goods delivered and services performed, and by shipping documents, and upon fulfillment of other obligations stipulated in the contract.

  - Payment will be made in the currency or currencies in which the Contract price has been stated in the Supplier’s bid as well as in other currencies in which the Supplier had indicated in its bid that it intends to incur expenditures in the performance of the Contract and wishes to be paid.

  - The payment will be made based on standard terms of international trade and the payment may be effected through irrevocable letters of credit drawn  through a Nationalized Bank or as decided by the Director of Fire and Rescue Services in case of a Foreign bidder.

- Taxes and Duties:

17.1.A foreign Supplier shall be entirely responsible for all taxes, stamp duties, license fees and other such levies imposed outside the Purchaser’s country.

17.2. A local supplier shall be entirely responsible for all taxes, duties, license fees, etc, incurred until delivery of the contracted goods to the Purchaser.

18. Validity of the offer:

18.1. The rate shall be valid for 180 days from the date of opening of the tenders.

18.2. Further, the tenderer shall agree to extend the validity of the tender without altering the substance and prices of their Bid for further periods, if any, required by the purchaser.

19. Reservation of Rights:

- While evaluating the technical bids If the offered models of the tendered Aerial Ladder Platform does not suit our conditions as laid down in technical specifications, the Director of Fire and Rescue Services reserves the right:

(1) to relax or waive or modify at his discretion any of the condition / specification in the interest of the Department

(2)  to reject all the Tenders  in respect of this item without assigning any reasons whatsoever and to suitably modify the numbers of Aerial Ladder Platforms to be bought of tendered items of Aerial Ladder Platforms to suit our requirement – consider and suitably modify

-0-

## FORM FOR TENDER AND CONDITION OF CONTRACT

Tender for the Supply of Articles.

To

The Governor of Tamil Nadu,

represented by

The  Director,

Office of the Director  of Fire & Rescue Services,

Egmore,  Chennai 600 008.

I,                 hereby contract and agree on the acceptance of this Tender   to  supply to the Government of  Tamil Nadu   ( hereinafter referred to as  “the Government” ) in accordance with the conditions of contract stated below, the articles specified in the schedule below of the quality of sort and at the rate of price therein specified and I hereby forward a sum of  Indian Rupee Value of Rs.12,00,000/- (Rupees Twelve lakhs only)  as Earnest Money Deposit  to be returned to me   if this tender is not accepted.

If this tender should be accepted, I hereby undertake to execute within the  date specified the department, a contract in accordance with the terms and conditions of contract aforesaid together with such further usual conditions as before the execution of the contract may be agreed upon between the successful Tender and the  authority concerned in the Department.

Signature of the Tenderer

with Date and Seal.

## CONDITION OF CONTRACT TO BE INCORPORATED IN THE AGREEMENT      WITH  SUCCESSFUL TENDERER

1.	This contract is to last for one year certain from its date.  But in the event of any breach of agreement at any time on the part of the contractor, the contract shall be terminated by the Director of Fire and Rescue Services, Chennai on behalf of the Governor of Tamilnadu upon giving fifteen days’ notice to the contractor.

2.	The articles to be supplied under this contract are to be of the quality or sort above mentioned and in every respect equal and answerable to the patterns or submitted during evaluation and such as the Director or any other duly authorised by him shall approve.

3.	The articles are to be delivered free of charge at the contractor’s risk in such quantities or number at such times and in such manner as the Director of Fire and Rescue Services, Chennai or anyone duly authorised by him shall from time to time order.

4.   Rejected articles shall be removed by and at the expense of the contractor within seven days after notice shall have been given him of the rejection.  If not so removed, the Director of Fire & Rescue Services, Chennai may cause the articles to be removed and charge the contractor with all expenses incurred in such removal.

5.	In case of failure by the contractor to deliver articles demanded from him within the period limited for delivery or in case of articles delivered by him not being the stipulated quality, sort, weight or measure or in case of articles being delivered without a correct invoice in triplicate, the Director of Fire & Rescue Services, Chennai or anyone duly authorised by him shall have power to reject any such articles and to purchase others  instead of  any articles so rejected or not delivered unless the contractor shall himself forthwith supply orders that shall be sufficient and satisfactory and any excess cost so incurred  by the Government over the contract price together with all charges and expenses attending the purchase shall be recoverable by the Government from the contactor.

6.	In case of failure to complete the supplies within the date of delivery specified in terms of the contract placed on the contractor the difference between his offer and the next selected offer shall be recoverable by the Government from the contractor.

7.	The contract or any part, share or interest in it is not to be transferred or assigned by the contractor directly or indirectly to any person or persons whomsoever without the written consent of the Director of Fire & Rescue Services, Chennai.

8.	The contractor shall make his own arrangements for obtaining import licences in respect of articles tendered for supply under this contact from the authorities concerned and in no case will his applications in this regard be forwarded.

9.	With every delivery of articles under the contract, the contractor should send invoices or bills in triplicate, which after verification will be passed for payment.

10.	The Government hereby agree that during the term fixed by this contract, and except as herein provided Government shall not purchase from any persons other than the contractor or from any Company or Corporation all or any quantities of the goods or materials agreed to be supplied by the contractor.

11.	Nothing herein contained shall prevent the Director of Fire and Rescue Services, Chennai from at any time requiring the contractor to supply and delivery any larger number of any one or more of the said articles than the number of the same mentioned in the said schedule and in any such case and so often as the same shall happen, the contractor shall and will supply and deliver as aforesaid such additional number of all or any of the said articles subject to the several provisions herein contained as shall be required by him and nothing herein contained shall render it obligatory on the Director of Fire and Rescue  Services, Chennai to take from the contractor the whole or any of the said articles and the contractor shall supply and deliver under this contract only such of the said articles as shall from time to time be required by or on behalf of the Director of Fire & Rescue Services, Chennai.

12.	Any notice to the contractor shall deemed to be sufficiently served if given or left in writing at his usual or last known place of abode or business.

13.	The  contractor shall furnish money security for the due fulfillment of the contract equivalent to                5 percent of the total value of the supplies contracted for.  This security may be in the form of Cash or Demand draft or Bankers’ cheque or specified small savings instruments or where the procuring entity deems fit, irrevocable bank guarantee in a prescribed form.

14.	Upon the complete fulfillment of this contract by the contractor to the satisfaction of the Government or any officer duly authorised by them, the security furnished by the contractor shall be returned to him, less the amount if any due by the contractor to the Government.

Noted the contents:

( Contractor’s  Signature )

GENERAL TERMS AND CONDITIONS FOR THE TENDER FOR THE SUPPLY OF THE TENDERED ITEMS TO FIRE AND RESCUE SERVICES DEPARTMENT.

# Tender Form

1.	The tender schedule is not transferable and only the tenderer to whom it is officially issued by the Fire and Rescue Services Department should use it.

2. The Vendors who have downloaded the Tender documents from the departmental website must intimate the same through the following e-mail address:  ddhq.tnfrs@gov.in,  asupt.tnfrs@gov.in  so that any changes can be intimated.  Otherwise, it will be considered that the vendor is doing it at his own risk.

3.	The tenderer has the option of sending the offer by Registered post or Speed Post or by courier or air mail  before the date and time fixed.

4.  The tenderers who could not send their tender by post can put the tender in the Tender Box kept for this purpose in person before the date and time fixed.  The slit of the tender box shall be 28 cm x 3 cm and therefore, the cover should be brought of suitable size to move inside the slit.  This Department will not be responsible for any postal or other transit delay in receipt of the tender offer.

5.    Telex/Fax or Telegraphic offers will not be entertained and will be rejected.

6.	Any offer received by the purchaser after the due date and time specified for submission of the tender offer will be summarily rejected and returned unopened to the tenderer.

7.	Only the original tender document supplied by this Office or the tender documents which have downloaded from the department’s website should be used and it should be resubmitted in full shape without detaching any page from it.  Xerox copies of copies typed on paper will not be accepted and the offer will be rejected.

8.   Incomplete tender documents will be rejected.

9.   Each page of the tender document, including technical specifications, should be duly

signed.

10. When the tender is made by an individual, it shall be signed with his name and address and in case of partnership firm, it shall be signed by one of the Partners empowered to submit tenders on behalf of the partnership as contemplated.  If it is made by a Company or Corporation it shall be signed by an officer duly authorised for the purpose.  In all cases, the status of the tenderer, whether a Proprietorship, Partnership, Company or Corporation should be furnished in the covering letter with full address of the tenderer's name, names of Partners, Directors etc., The tenderer should also furnish such particulars / evidence  as may be required by the competent authority in this Department before execution of the agreement.

11.	If any covering letter accompanying the tender document is in the tenderers letter head form, all terms and conditions printed therein should be scored out and duly authenticated.                    It should be distinctly understood that such printed terms and conditions in the letter head will not have any binding on this Department and those terms and conditions quoted in the tender document only will be considered.

12.	No erasures, interpolations or  alterations will be allowed except when initialed  by the tenderer.

13.	Only manufacturers / their authorised distributors/ dealers can participate in the tender. The authorized distributor/dealer should produce the specific authorization from the original equipment manufacturer.

14. The vendor if he wishes, may, apart from the answers to the questionnaire enclose brochures and leaflets.

# Earnest Money Deposit

15. Earnest Money Deposit amount  in Indian Rupee value as prescribed in the top sheet of the tender document should be paid through their Indian Representative along with tender by way of Demand Draft in favour of the Director of Fire & Rescue Services  Department, Chennai should be compulsorily appended with the tender document.  Specified small savings instruments or irrevocable bank guarantee will also be accepted.  Personal / Company cheques will not be accepted.

16.	Any tender submitted without EMD and partial payment of EMD will be summarily rejected.

17.  In case of unsuccessful tenderers, their Earnest Money Deposit will be returned to them in due course soon after the final decision is taken on the tenders by the department.

18.  The Earnest Money Deposit will not bear any interest.

# Prices:  Details to be furnished and conditions

19.   Rates should be shown distinctly in words as well as figures.

20. Prices quoted should include the cost of all accessories essential to make the equipment / article fully functional.

21.	While giving the Price, the details like basic rate (i.e. the rate without taxes and duties) along with separate details of GST, Custom duty, Excise duty, freight charges, Octroi, insurance for delivery and such other levies that may be applicable   must be quoted only  in the price bid format attached with tender document.    The total of all these must be separately indicated as NET RATE. Rates should be shown distinctly in words as well as figures. In case of a discrepancy between the rate in words and figures,  the lower of the two shall be taken.   The rates given in any forms other than the  price bid format attached with the tender document will not be considered  and will be rejected as non-responsive.

22.  The rate shall be valid for 180 days from the date of opening of the tenders.

23.	Further, the tenderer shall agree to extend the validity of the tender without altering the substance and prices of their Bid for further periods, if any, required by the purchaser.

24.	No charges other than those mentioned in the supply order will be paid under any circumstances.

# Verification/Demonstration of equipment and Evaluation

25.	The evaluation of tenders will be made as per “ Stage III- Criteria for Evaluation” given in the tender document.

26. The tenderer should produce the equipment / materials which he has quoted for providing in the Aerial Ladder Platform for demonstration before the evaluation committee as and when called for.   The tender will be rejected if the article is not produced for demonstration  the Earnest Money Deposit is liable to be forfeited in the event of such failure.

27.	Each sample produced for demonstration must have a label attached to it, showing clearly the name of the tenderer and the country of manufacture.

# Warranty

28.	The warranty period shall be as may be stipulated for the article concerned from the date of supply.

29.	The warranty shall cover defects in materials or bad workmanship under normal use and service for the period of warranty notwithstanding the fact that the material were inspected and approved by inspecting officers.

# Acceptance of tender

30. The lowest tender will not necessarily be accepted.

31.	Notification of acceptance of tender by the Department shall be deemed to result in and constitute a valid and considered contract binding on the tenderer.  If the tenderer withdraws  his / her tender after acceptance by the Department, the EMD shall be forfeited.

Release of Purchase Order.

32.  The contractor should examine carefully the purchase order immediately on receipt and bring to the notice of the indenting officer within five days of receipt of purchase order and discrepancy with regard to the quantity ordered, specifications, packing, etc., for prompt rectification. The letter should be sent by registered post Acknowledgement due.  Meanwhile, supply of the materials, which are cleared in all respects for despatch and acceptance by the consignees shall be arranged by the date mutually, agreed upon and should not be held up for any reason whatsoever.

# Agreement

33.	The successful tenderer should execute an agreement in non-judicial stamp paper as specified while placing provisional order of acceptance by the Department for the due fulfillment of the contract.

34.	Successful tenderers will be required to execute an agreement in the Form, which can be seen at the Office of the Director of Fire & Rescue Services, Chennai.   The Stamp duty on the agreement shall be borne by the successful tenderer.   On their executing the agreement, the Earnest Money will be returned to them.  The charges for refunding the money by way of Demand Draft  will be deducted from the amount refundable

35.	If the tenderer is from abroad, payment terms for imports shall be based on standard terms of international trade and the payment will be effected through irrecoverable letters of credit drawn on banks.

# Security Deposit

36.	Security Deposit equivalent to 5 percent of the total value of the supplies contracted for will have to be furnished by the successful tenderer through their representative in India.      This security may be paid in the form of Cash or Demand draft or Bankers’ cheque or specified small savings instruments or where the procuring entity deems fit, irrevocable bank guarantee in a prescribed form.    This is liable to be forfeited either in full or in part for any breach of conditions of contract or default on the part of the contractor.

37.	The Earnest Money Deposit will be forfeited if the successful tenderer fails to  remit the Security Deposit and execute the agreement within the period specified.  The security deposit will be refunded only upon satisfactory fulfillment of the contract after completion of supply of the entire materials ordered and expiry of the guarantee period duly adjusting the amount due to the Department if any.  However, if the tenderer has been insisted for  Performance Guarantee for certain items as per the discretion of the DFRS, the Security Deposit will be refunded to the tenderer.

38.  Security Deposit paid by the successful tenderer will not bear any interest.

39.	On successful completion of supply to the satisfaction of the Fire & Rescue Services Department and on receipt of the invoice bill in triplicate, 100% payment will be settled immediately.

40.	The Director of Fire & Rescue Services Department, Chennai, acting on behalf of the Governor of Tamil Nadu or any other authority empowered in this matter shall have the right of rejecting any tender without assigning any reason for doing so.

41.	The proposed quantity to be procured in respect of each item is mentioned in the tender document in relevant page.   However, the Director of Fire & Rescue Services Department, Chennai or any other authority empowered shall have the right or requiring a successful tenderer to supply a greater number or reduce the number than the number or quantity mentioned in the schedule

42. Tenders will be liable to rejection for the following reasons apart from any other reasons mentioned anywhere else in this document.

  - If the country of manufacture and of the origin of material used in manufacture of articles is not noted in the tender.

  - If the tender is not in the form supplied by this Office or downloaded from the departmental website.

  - If the offer is not as per the specifications mentioned in this tender document.

  - If samples of / models of articles quoted are not submitted as and when required for technical evaluation test.

  - If the tender is not accompanied by the receipt of Earnest Money.

  - If the tenderer enters any restriction on other conditions in his tender.

  - If the tenderer enters one rate in figures and another in words and declines to abide by the lower of the two.

  - If the tenderer alters the period of supply or expunges any clause in the form of tender, and

  - If the tender is presented without signature.

# Reservation of rights

43.  The authority competent reserves the rights:-

(1) The Tender Inviting Authority may extend the last date and time for receiving tenders after giving adequate notice to all intending tenderers in cases where:-

- the publication of the tender notice has been delayed

- the communication of changes, in the tender documents to the prospective tenderers took time

- any of the tenderers requested clarifications communication of which took time to all the tenderers and

- any other reasonable grounds exist, for such extension which shall be recorded in writing by the Tender Inviting Authority.

(2) If at any stage, a technical flaw is noticed in following the procedures, the  procuring entity will take appropriate decisions to ensure that the overall objective of procurement is not defeated.

(3) After negotiation with the tenderer and before passing the order accepting a tender,               if the Tender Accepting Authority decides that the price quoted by such tenderer is higher by the percentage as may be prescribed over the schedule of rates or prevailing market rates, the tender shall be rejected.

(4) The Tender Accepting Authority before passing the order accepting a tender, may also reject all the tenders for the reasons such as changes in the scope of procurement, lack of anticipated financial resources, court orders, accidents or calamities and other unforeseen circumstances.

(5) The purchaser reserves the right to:-

  - vary the quantity finally ordered to the extent of 25% indicated in the Tender document.

  - To split the tender quantity and place orders on one or more than one firm to meet the delivery requirements.

  - Recover losses, if any, sustained by this Department, from the supplier who pleads his inability to supply and backs out of his obligation after award of contract.   The EMD and the Security Deposit paid shall be forfeited.

  - To cancel the orders for not keeping up the delivery schedule.

(v)    To award contract to one or more tenderers in respect of each item covered by the tender.

(vi)   To reject any or all the tenders without assigning any reasons therefor.

(vii) To relax or waive or modify at his discretion any of the condition / specification in the interest of the Department.

(viii) The decision of the tender accepting authority  in this regard shall be final, conclusive and binding on the supplier.

# Delivery

44.	The rates accepted will be for delivery at such destinations as indicated in the tender document or in subsequent purchase order.

45.	The delivery period should be mentioned by the Tenderers in their offers.  However, the maximum time limit for supply of the goods will be fixed by the Director while executing the agreement with the successful tenderer. The supplier shall reckon the delivery period from the date of receipt of the purchase order.  The date of delivery as per postal acknowledgement due in the case of purchase order sent by post or the date of delivery if delivered in person or through special messenger shall be deemed to be the date of receipt of the purchase ordered

46.	Delivery period is inclusive of pre-delivery inspection.  The date if delivery of materials in good condition to the consignees shall be the date of supply.  The rate of progress indicated in the purchase order should be maintained.  The materials should be delivered to the consignees as per the purchase order placed.

47.	Acceptance of materials by the consignee:-   If on examination of the sample, any portion of the supplied materials / equipment is found to be damaged or substandard or not fully in accordance with the relevant specifications, the whole supply  shall be rejected irrespective of the fact that this material was cleared under pre-delivery inspection.  The materials / equipment should strictly conform to the specifications given in the tender conditions.  In case of doubt whether the materials conform to the specifications or not, the decision of the consignee shall be final.  The consignee has right to reject the equipment found not conforming to the specifications and they should be replaced within seven days at the cost of the supplier.

# Penalty: -

48. Where their supply is not completed within the delivery schedule, without prejudice to the right of the Department to cancel undelivered portion of the purchase order, extension of time may, if so required by the contractor may be granted at the discretion of this Department.  Such extension of time will entail recovery of penalty from the contractor at one percent per week, on the value of the undelivered quantities subject to a maximum of five percent of the value of the undelivered quantity.  However, the Director may waive the penalty on valid reasons as per his discretion.

49.	The officer authorised by this Department will have the right to cancel the purchase order and to recover from the contractor liquidated damages at 2% of the value of the undelivered quantity or the extra financial commitment to this Department on the purchase of the balance quantity, whichever is higher besides forfeiting the security deposit.

Performance guarantee

50. 10 percent of the   payment amount will be retained if necessary as per the discretion of the DFRS and as per agreement during the warranty period as performance guarantee and will be paid after successful completion of the warranty depends upon the items.  Alternatively, if the tenderer gives an irrecoverable Bank Guarantee in lieu of the balance 10 percent through their representative in India the department reserves the right to accept the same.

# Certificates and Details to be furnished

51.	Latest tax verification certificate applicable to the Country/origin of the Supplier should also be appended with the tender document.

52.	The tenderer should furnish details of orders executed for Government Departments / Departments / autonomous boards etc., and details of orders on hand or pending execution.  Certificates in proof of these should also be furnished.

## CERTIFICATE BY THE TENDERER

I / we have gone through the terms and conditions and will abide by them as laid down above and in other sections of this Tender document.

I / We hereby declare that the particulars furnished by us in this offer are true to the best of my / our knowledge and we understand accept that, if at any stage the information furnished are found to be incorrect or false, we are liable for disqualification from this tender and also liable for any penal action that may arise due to the above.

Signature of the Tenderer

with Date and Seal.

| FORMAT PRICE BID FOR SUPPLY OF “Aerial Ladder Platform of not less than 54 m” ( Note: To be furnished in “ ENVELOPE –B” ) (The Price bid for supply of Aerial Ladder Platform not less than 54 meters should include the cost for supply of tools and accessories mentioned in the specification. For different makes/models should be furnished in a separate sealed cover) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No | i) Description of article ii) Make and Model iii)capacity iv) Country or origin of the article or components | Basic Price Outside the Purchaser’s country, the prices shall be quoted either U.S. Dollars or in the freely convertible currency. | Custom duty, Other taxes,levies, charges, clearance charges from Port and transportation charges etc. (specify the type of tax/charge and the amount for each such item) | GST or any other similar type of tax applicable at the supplier’s place | Total including Taxes, levies, charges including Customs duty, clearance charges from Port and transportation charges etc. | Qty. | Grand total for entire quantity | Other details (FOR terms, Payment terms, Validity, Delivery Schedule) |

Note: Additional sheet may be attached if necessary for furnishing the rates for different models.

Signature of the Tenderer with seal